Lab Report No 11

Demonstration of Matrix Keyboard Control using 8051 Interrupts.

Embedded Systems

Submitted By:

[ Huzaifa Shahbaz]

[21-CE-009]

[Aqsa Shah]

[21-CE-039]

Submitted to:

Engr. Bushra Fiaz

Dated:

Week 13

Department of Computer Engineering,

HITEC University, Taxila

Lab Task No 2:

## Solution:

Brief description (3-5 lines)

Write a program which generate the following patterns on LED bar. a) If button 0 is pressed = >; Turn on LED bar from left to right b) If button 1 is pressed =>; Turn on LED bar from right to left

The code

ORG 0000H LJMP MAIN ORG 0030H MAIN: MOV P1, #0FFH ; Configure P1 as input for buttons MOV P2, #00H ; Configure P2 as output for LED bar CHECK_BUTTONS: MOV A, P1 ; Read button states ; Check if button 0 is pressed (active low) JNB P1.0, LEFT_TO_RIGHT ; Check if button 1 is pressed (active low) JNB P1.1, RIGHT_TO_LEFT SJMP CHECK_BUTTONS LEFT_TO_RIGHT: MOV R0, #8 ; Counter for 8 LEDs MOV R1, #80H ; Start with leftmost LED (10000000B) L2R_LOOP: MOV P2, R1 ; Output pattern to LED bar LCALL DELAY ; Add delay MOV A, R1 ; Move current pattern to accumulator RR A ; Rotate right MOV R1, A ; Store back to R1 DJNZ R0, L2R_LOOP ; Repeat until all LEDs are lit MOV P2, #0FFH ; Turn on all LEDs LCALL DELAY MOV P2, #00H ; Turn off all LEDs SJMP CHECK_BUTTONS RIGHT_TO_LEFT: MOV R0, #8 ; Counter for 8 LEDs MOV R1, #01H ; Start with rightmost LED (00000001B) R2L_LOOP: MOV P2, R1 ; Output pattern to LED bar LCALL DELAY ; Add delay MOV A, R1 ; Move current pattern to accumulator RL A ; Rotate left MOV R1, A ; Store back to R1 DJNZ R0, R2L_LOOP ; Repeat until all LEDs are lit MOV P2, #0FFH ; Turn on all LEDs LCALL DELAY MOV P2, #00H ; Turn off all LEDs SJMP CHECK_BUTTONS DELAY: ; Delay subroutine MOV R6, #5 DELAY_OUTER: MOV R7, #200 DELAY_INNER: DJNZ R7, DELAY_INNER DJNZ R6, DELAY_OUTER RET END

The results (Screenshot)

Conclusion

In conclusion, the 8051 microcontroller was used in a demonstration of interrupt-based matrix keyboard control, demonstrating its efficiency and reliability. Key presses were detected through hardware interrupts, eliminating the need for continuous polling. The matrix keyboard interface was reliable for user input, and the interrupt service routine effectively handled key scanning and decoding, proving the power of interrupt-driven I/O.