Lab Report No _12_

Understanding and practice of Timer/Counter Control of 8051.

Embedded Systems

Submitted By:

[Huzaifa Shahbaz]

[21-CE-009]

[Aqsa Shah]

[21-CE-039]

Submitted to:

Engr. Bushra Fiaz

Dated:

Week 13

Department of Computer Engineering,

HITEC University, Taxila

Lab Task No 1:

## Solution:

Brief description (3-5 lines)

Design the Digital clock in such a way that it appears in LCM screen on 00:00:00 (hours:minutes:seconds) format. Delay Programming in msec, sec and min is further broken down into some function which is called after a specific time interval. Update screen function is invoked every 1 second to update the LCD display. 8051 Timers have been applied to achieve 1 msec accurate delay. Pressing key "1" starts the clock or resumes; pressing key "2" pauses counting and pressing key "2" again resets the clock.

The code

#include <reg51.h> #include <string.h> // LCD pin definitions sbit RS = P2^0; sbit EN = P2^1; sbit KEY1 = P1^0; // Start/Resume key sbit KEY2 = P1^1; // Pause/Reset key // Global variables for time unsigned char seconds = 0; unsigned char minutes = 0; unsigned char hours = 0; unsigned char running = 0; unsigned int ms_count = 0; // Function prototypes void init_timer0(void); void delay_ms(unsigned int ms); void lcd_cmd(unsigned char cmd); void lcd_data(unsigned char dat); void lcd_init(void); void lcd_string(unsigned char *str); void update_display(void); void check_keys(void); // Timer0 ISR void timer0_isr(void) interrupt 1 { TH0 = 0xFC; // Reload timer for 1ms delay TL0 = 0x66; ms_count++; if(ms_count >= 1000 && running) { // 1 second has passed ms_count = 0; seconds++; if(seconds >= 60) { seconds = 0; minutes++; if(minutes >= 60) { minutes = 0; hours++; if(hours >= 24) { hours = 0; } } } update_display(); } } // Initialize Timer0 void init_timer0(void) { TMOD = 0x01; // Timer0 in mode 1 (16-bit) TH0 = 0xFC; // Initial values for 1ms delay TL0 = 0x66; ET0 = 1; // Enable Timer0 interrupt EA = 1; // Enable global interrupts TR0 = 1; // Start Timer0 } // Millisecond delay function void delay_ms(unsigned int ms) { unsigned int i; for(i = 0; i < ms; i++) { TH0 = 0xFC; // 1ms delay values TL0 = 0x66; TR0 = 1; while(!TF0); TR0 = 0; TF0 = 0; } } // LCD functions void lcd_cmd(unsigned char cmd) { P0 = cmd; RS = 0; EN = 1; delay_ms(5); EN = 0; } void lcd_data(unsigned char dat) { P0 = dat; RS = 1; EN = 1; delay_ms(5); EN = 0; } void lcd_init(void) { lcd_cmd(0x38); // 2 lines, 5x7 matrix lcd_cmd(0x0C); // Display ON, cursor OFF lcd_cmd(0x01); // Clear display lcd_cmd(0x80); // Move cursor to beginning of first line } void lcd_string(unsigned char *str) { while(*str) { lcd_data(*str++); } } // Update LCD display void update_display(void) { unsigned char time_str[9]; lcd_cmd(0x85); // Position cursor at center of first line // Format time string time_str[0] = (hours/10) + '0'; time_str[1] = (hours%10) + '0'; time_str[2] = ':'; time_str[3] = (minutes/10) + '0'; time_str[4] = (minutes%10) + '0'; time_str[5] = ':'; time_str[6] = (seconds/10) + '0'; time_str[7] = (seconds%10) + '0'; time_str[8] = '\0'; lcd_string(time_str); } // Check key inputs void check_keys(void) { if(!KEY1) { // Start/Resume delay_ms(20); // Debounce if(!KEY1) { running = 1; while(!KEY1); // Wait for key release } } if(!KEY2) { // Pause/Reset delay_ms(20); // Debounce if(!KEY2) { if(running) { // If running, pause first running = 0; } else { // If already paused, reset hours = 0; minutes = 0; seconds = 0; update_display(); } while(!KEY2); // Wait for key release } } } // Main function void main(void) { // Initialize LCD and Timer lcd_init(); init_timer0(); // Display initial time update_display(); // Main loop while(1) { check_keys(); } }

The results (Screenshot)

Conclusion

In conclusion, we had done and performed the task practice timer/counter. In this lab task, we understood the program of digital clock by doing the 8051 Keil C language and then we implement in the flash magic by doing HEX file.