Lab Report No 13

Understanding and practice of LCM display control to print various messages.

Embedded Systems

Submitted By:

[Huzaifa Shahbaz]

[21-CE-009]

[Aqsa Shah]

[21-CE-039]

Submitted to:

Engr. Bushra Fiaz

Dated:

Week 16

Department of Computer Engineering,

HITEC University, Taxila

Lab Task No 1:

## Solution:

Brief description (3-5 lines)

Write a program which Show the character string “Hello ! Welcome” in the middle of the first line of LCM and blink it every 0.5 second interval.

The code

ORG 0000H ; Start of the code at address 0000H MOV SP, #60 CLR P1.7 CALL LCMINIT MOV A, #10000000B CALL WRINS MOV DPTR, #LINE CALL PRTSTR MAIN_LOOP: CALL PRTSTR CALL DELAY_500MS CLR P1.5 CLR P1.6 CLR P1.7 CALL DELAY_500MS JMP MAIN_LOOP WRINS: CALL CHKBSY CLR P1.5 CLR P1.6 SETB P1.7 MOV P0, A CLR P1.7 RET WRDATA: CALL CHKBSY SETB P1.5 CLR P1.6 SETB P1.7 MOV P0, A CLR P1.7 RET CHKBSY: MOV P0, #0FFH CLR P1.5 SETB P1.6 SETB P1.7 MOV C, P0.7 CLR P1.7 JC CHKBSY RET LCMINIT: MOV A, #00110000B CALL WRINS MOV A, #00000001B CALL WRINS MOV A, #00001100B CALL WRINS MOV A, #00000110B CALL WRINS RET PRTSTR: MOV A, #0 MOVC A, @A + DPTR CJNE A, #'$', PRINT RET PRINT: CALL WRDATA INC DPTR JMP PRTSTR DELAY_500MS: MOV R6, #50 DELAY_LOOP_500MS: MOV R7, #200 DELAY_LOOP_200MS: DJNZ R7, DELAY_LOOP_200MS DJNZ R6, DELAY_LOOP_500MS RET LINE: DB 'Hello! Welcome$' end

The results (Screenshot)

Conclusion

In conclusion, we had performed the LCM display control to print various messages. In the assembly language to print hello and welcome string in the Keil 8051 and its show in flash magic.