Lab Report No _5_

Led Pattern generation using EdSim51 Simulator

Embedded Systems

Submitted By:

[Huzaifa Shahbaz]

[21-CE-009]

[Aqsa Shah]

[21-CE-039]

Submitted to:

Engr. Bushra Fiaz

Dated:

Week 05

Department of Computer Engineering,

HITEC University, Taxila

Lab Task No 1:

## Solution:

Brief description (3-5 lines)

Write a program which act as up and down counter from 0 to 9 and 9 to 0 using the 8- LEDs.

Code

ORG 0000H LJMP MAIN ORG 0030H MAIN: MOV P1, #0FFH ; Initialize Port 1 as output MOV R0, #0 ; Counter value MOV R1, #0 ; Direction flag (0=up, 1=down) LOOP: MOV A, R0 ; Move counter value to accumulator CALL DISPLAY ; Convert and display number CALL DELAY ; Add delay MOV A, R1 ; Check direction JZ COUNT_UP ; If R1=0, count up COUNT_DOWN: DJNZ R0, LOOP ; Decrement and continue if not zero MOV R1, #0 ; Change direction to up MOV R0, #0 ; Reset counter to 0 SJMP LOOP COUNT_UP: INC R0 ; Increment counter CJNE R0, #10, LOOP ; Continue if not 10 MOV R1, #1 ; Change direction to down MOV R0, #9 ; Set counter to 9 SJMP LOOP DISPLAY: MOV DPTR, #PATTERN ; Load pattern table address MOVC A, @A+DPTR ; Get pattern for current number MOV P1, A ; Output to LEDs RET DELAY: MOV R6, #5 ; Outer loop counter DELAY1: MOV R7, #200 ; Inner loop counter DELAY2: DJNZ R7, DELAY2 ; Decrement inner counter DJNZ R6, DELAY1 ; Decrement outer counter RET PATTERN: ; LED patterns for numbers 0-9 DB 11000000B ; 0 DB 11111001B ; 1 DB 10100100B ; 2 DB 10110000B ; 3 DB 10011001B ; 4 DB 10010010B ; 5 DB 10000010B ; 6 DB 11111000B ; 7 DB 10000000B ; 8 DB 10010000B ; 9 END

The results (Screenshot)

Lab Task No 2:

## Solution:

Brief description (3-5 lines)

Write a program which turn on the rightmost led by pressing the leftmost switch and so on.

Code

ORG 0H SJMP MAIN ORG 30H MAIN: ; Initialize Port 1 for LEDs (Output) MOV P1, #0FFH ; Main program loop LOOP: ; Read switches from P0 MOV A, P0 ; Reverse the bits using rotate operations MOV B, #08H ; Counter for 8 bits MOV R0, #00H ; Initialize result register REVERSE: RRC A ; Rotate right through carry MOV C, ACC.0 ; Get the bit that was rotated MOV A, R0 ; Get the result so far RLC A ; Rotate left through carry MOV R0, A ; Store the result MOV A, B ; Get the counter DEC A ; Decrement counter MOV B, A ; Store the counter MOV A, P0 ; Get the original input again JNZ REVERSE ; If not done, continue reversing ; Output reversed value to LEDs MOV A, R0 ; Get the reversed result CPL A ; Complement (LEDs are active-low) MOV P1, A ; Output to LEDs SJMP LOOP ; Repeat forever END

The results (Screenshot)

Lab Task No 3:

## Solution:

Brief description (3-5 lines)

Write a program code which shows the number of key pressed on LED bar.

Code

ORG 0H LJMP MAIN ORG 100H MAIN: ; Initialize counter in R0 MOV R0, #0 ; R0 will store the count of key presses ; Configure Port 1 for LEDs (output) MOV P1, #0FFH ; Initially all LEDs are OFF LOOP: ; Check if key is pressed (P2.0) JB P2.0, CHECK_RELEASE ; If key not pressed (P2.0 = 1), check for release SJMP LOOP ; If still pressed, keep checking CHECK_RELEASE: ; Wait for key release JNB P2.0, LOOP ; If key pressed (P2.0 = 0), go back to loop ; Key was released, increment counter INC R0 ; Increment the counter ; Check if counter exceeded 8 (max LEDs) MOV A, R0 CJNE A, #9, DISPLAY ; If not 9, continue to display MOV R0, #0 ; If 9, reset counter to 0 DISPLAY: ; Convert counter to LED pattern MOV A, R0 ; Move counter to accumulator MOV B, #0FFH ; Start with all LEDs off ; Skip if counter is 0 JZ SHOW_LEDS CONVERT_LOOP: RR A ; Rotate right (will set carry if 1) MOV B, A ; Move pattern to B CPL B ; Complement B (0s become 1s and vice versa) DJNZ ACC, CONVERT_LOOP ; Decrement A and continue if not zero SHOW_LEDS: MOV P1, B ; Output pattern to LEDs ; Add delay for debouncing ACALL DELAY SJMP LOOP ; Return to main loop DELAY: MOV R7, #200 DELAY_OUTER: MOV R6, #250 DELAY_INNER: DJNZ R6, DELAY_INNER DJNZ R7, DELAY_OUTER RET END

The results (Screenshot)

Conclusion

In conclusion, The EdSim51 Simulator was used to generate LED patterns, demonstrating the fundamental principles of 8051 microcontroller programming and I/O operations. The project demonstrated bit manipulation and delay routines, showcasing the simplicity and effectiveness of basic microcontroller concepts. This hands-on experience provides valuable insights into embedded systems programming and real-time pattern generation, laying a strong foundation for more complex microcontroller applications.