Lab Report No 8

Implementation of Color Models and Color Image Processing

Digital Image Processing

Submitted By:

[Huzaifa Shahbaz]

[21-CE-009]

[Aqsa Shah]

[21-CE-039]

Submitted to:

Engr. Hareem Khan

Dated:

29thNov,2024

Department of Computer Engineering,

HITEC University, Taxila

Lab Example No 1:

## Solution:

Brief description (3-5 lines)

This is an additive model, i.e. the colours present in the light add to form new colours, and is appropriate for the mixing of coloured light for example. The image shows the additive mixing of red, green and blue primaries to form the three secondary colours yellow (red + green), cyan (blue + green) and magenta (red + blue), and white ((red + green + blue).

The code

import cv2 import matplotlib.pyplot as plt # Load the image image = cv2.imread('input_image.jpg') image_rgb = cv2.cvtColor(image, cv2.COLOR_BGR2RGB) # Convert to RGB for display # Split the image into R, G, B channels r_channel, g_channel, b_channel = cv2.split(image_rgb) # Display the original image and each channel plt.figure(figsize=(12, 6)) # Original Image plt.subplot(1, 4, 1) plt.imshow(image_rgb) plt.title("Original RGB Image") plt.axis('off') # Red Channel plt.subplot(1, 4, 2) plt.imshow(r_channel, cmap='Reds') plt.title("Red Channel") plt.axis('off') # Green Channel plt.subplot(1, 4, 3) plt.imshow(g_channel, cmap='Greens') plt.title("Green Channel") plt.axis('off') # Blue Channel plt.subplot(1, 4, 4) plt.imshow(b_channel, cmap='Blues') plt.title("Blue Channel") plt.axis('off') plt.tight_layout() plt.show()

The results (Screenshot)

Lab Example No 2:

## Solution:

Brief description (3-5 lines)

The CMY (cyan-magenta-yellow) model is a subtractive model appropriate to absorption of colours, for example due to pigments in paints. Whereas the RGB model asks what is added to black to get a particular colour, the CMY model asks what is subtracted from white. In this case,the primaries are cyan, magenta and yellow, with red, green and blue as secondary colours

The code

import cv2 import numpy as np import matplotlib.pyplot as plt # Read the RGB image I = cv2.imread('The CMY Model.jpeg') I = cv2.cvtColor(I, cv2.COLOR_BGR2RGB) # Convert BGR to RGB for proper color display # Display the original RGB image plt.figure() plt.imshow(I) plt.title("Original RGB Image") plt.axis('off') # Create a CMY image by subtracting each channel from 255 CMY = np.zeros_like(I) # Initialize CMY array with the same shape as I R = I[:, :, 0] # Red channel G = I[:, :, 1] # Green channel B = I[:, :, 2] # Blue channel # Compute CMY channels CMY[:, :, 0] = 255 - R # Cyan (complement of red) CMY[:, :, 1] = 255 - G # Magenta (complement of green) CMY[:, :, 2] = 255 - B # Yellow (complement of blue) # Display the CMY image plt.figure() plt.imshow(CMY) plt.title("CMY Image") plt.axis('off') plt.show()

The results (Screenshot)

Lab Example No 2:

## Solution:

Brief description (3-5 lines)

As mentioned above, colour may be specified by the three quantities hue, saturation and intensity.This is the HSI model, and the entire space of colours that may be specified in this way showing the HSI solid on the left, and the HSI triangle on the right, formed by taking a horizontal slice through the HSI solid at a particular intensity. Hue is measured from red, and saturation is given by distance from the axis. Colours on the surface of the solid are fully saturated, i.e. pure colours, and the greyscale spectrum is on the axis of the solid. For these colours, hue is undefined.

The code

import cv2 import numpy as np import matplotlib.pyplot as plt # Read the RGB image and normalize it to [0, 1] Img = cv2.imread('import cv2') import numpy as np import matplotlib.pyplot as plt # Read the RGB image and normalize it to [0, 1] Img = cv2.imread('The HSI Model.jpg') Img = cv2.cvtColor(Img, cv2.COLOR_BGR2RGB) # Convert BGR to RGB Img = Img.astype(np.float32) / 255.0 # Normalize to range [0, 1] # Display the original image plt.figure() plt.imshow(Img) plt.title("Original RGB Image") plt.axis('off') Img = cv2.cvtColor(Img, cv2.COLOR_BGR2RGB) # Convert BGR to RGB Img = Img.astype(np.float32) / 255.0 # Normalize to range [0, 1] # Display the original image plt.figure() plt.imshow(Img) plt.title("Original RGB Image") plt.axis('off') # Extract R, G, B channels R = Img[:, :, 0] G = Img[:, :, 1] B = Img[:, :, 2] # Hue calculation numerator = 0.5 * (R - G + R - B) denominator = np.sqrt((R - G) ** 2 + (R - B) * (G - B)) + 1e-7 # Avoid division by zero H = np.arccos(numerator / denominator) # Adjust H where B > G H[B > G] = 2 * np.pi - H[B > G] H = H / (2 * np.pi) # Normalize to range [0, 1] # Intensity calculation I = (R + G + B) / 3 # Saturation calculation minRGB = np.minimum(np.minimum(R, G), B) S = 1 - 3 * minRGB / (R + G + B + 1e-7) # Avoid division by zero # Combine H, S, and I into one visualization HSI = np.stack((H, S, I), axis=-1) # Display H, S, and I components plt.figure(figsize=(12, 4)) # Hue plt.subplot(1, 3, 1) plt.imshow(H, cmap='hsv') plt.title("Hue") plt.axis('off') # Saturation plt.subplot(1, 3, 2) plt.imshow(S, cmap='gray') plt.title("Saturation") plt.axis('off') # Intensity plt.subplot(1, 3, 3) plt.imshow(I, cmap='gray') plt.title("Intensity") plt.axis('off') plt.tight_layout() plt.show()

The results (Screenshot)

Lab Task No 1:

## Solution:

Brief description (3-5 lines)

Convert CMY to RGB with key color 0.412 where the formula is [R G B] = 255 ∗ [1 – C 1 – M 1 − Y] ∗ [1 – key 1 – key 1 − key]

The code

import cv2 import numpy as np import matplotlib.pyplot as plt # Import the image img = plt.imread("The CMY Model.jpg") # Normalize the BGR image to range [0, 1] bgr = img.astype(float) / 255.0 # Calculate CMYK channels with np.errstate(invalid='ignore', divide='ignore'): K = 1 - np.max(bgr, axis=2) C = (1 - bgr[..., 2] - K) / (1 - K + 1e-10) # Add a small value to avoid division by zero M = (1 - bgr[..., 1] - K) / (1 - K + 1e-10) Y = (1 - bgr[..., 0] - K) / (1 - K + 1e-10) # Stack the channels and scale to [0, 255] CMYK = (np.dstack((C, M, Y, K)) * 255).astype(np.uint8) # Split CMYK channels C, M, Y, K = cv2.split(CMYK) # Save each CMYK channel as a separate image cv2.imwrite('C_Channel.jpg', C) cv2.imwrite('M_Channel.jpg', M) cv2.imwrite('Y_Channel.jpg', Y) cv2.imwrite('K_Channel.jpg', K) # Function to convert CMY to RGB def cmy_to_rgb(c, m, y, key): r = (1 - c) * (1 - key) g = (1 - m) * (1 - key) b = (1 - y) * (1 - key) return r, g, b # Example usage of the CMY to RGB conversion function c = 1 - 0.412 m = 1 - 0.412 y = 1 - 0.412 key = 0.412 r, g, b = cmy_to_rgb(c, m, y, key) # Print the RGB values print(f"R: {r:.3f}") print(f"G: {g:.3f}") print(f"B: {b:.3f}")

The results (Screenshot)

K_Channel M_Channel Y_Channel

Lab Task No 2:

## Solution:

Brief description (3-5 lines)

Extract blue color letters from the given image where the color range is (100,0,0) to (130,255,255)

The code

import cv2 import numpy as np # Load the image image = cv2.imread('marketing.jpg') # Convert the image from BGR to HSV color space hsv_image = cv2.cvtColor(image, cv2.COLOR_BGR2HSV) # Define the lower and upper bounds for the blue color in HSV lower_blue = np.array([100, 0, 0]) upper_blue = np.array([130, 255, 255]) # Create a mask using the defined color range mask = cv2.inRange(hsv_image, lower_blue, upper_blue) # Bitwise-AND mask and original image to extract blue areas extracted_blue = cv2.bitwise_and(image, image, mask=mask) # Display the original image and the extracted blue areas cv2.imshow('Original Image', image) cv2.imshow('Extracted Blue Letters', extracted_blue) # Wait for a key press and close the image windows cv2.waitKey(0) cv2.destroyAllWindows()

The results (Screenshot)

Lab Task No 3:

## Solution:

Brief description (3-5 lines)

Apply HSV and RAINBOW color map to the gray scale image.

The code

import cv2 import matplotlib.pyplot as plt import numpy as np # Load the grayscale image image_path = 'grayscale.jpg' # Replace with the path to your image grayscale_image = cv2.imread(image_path, cv2.IMREAD_GRAYSCALE) # Apply HSV and RAINBOW colormaps hsv_colormap = cv2.applyColorMap(grayscale_image, cv2.COLORMAP_HSV) rainbow_colormap = cv2.applyColorMap(grayscale_image, cv2.COLORMAP_RAINBOW) # Display the results plt.figure(figsize=(15, 5)) # Original grayscale image plt.subplot(1, 3, 1) plt.title("Grayscale Image") plt.imshow(grayscale_image, cmap='gray') plt.axis('off') # HSV colormap plt.subplot(1, 3, 2) plt.title("HSV Colormap") plt.imshow(cv2.cvtColor(hsv_colormap, cv2.COLOR_BGR2RGB)) plt.axis('off') # RAINBOW colormap plt.subplot(1, 3, 3) plt.title("RAINBOW Colormap") plt.imshow(cv2.cvtColor(rainbow_colormap, cv2.COLOR_BGR2RGB)) plt.axis('off') plt.tight_layout() plt.show()

The results (Screenshot)

Conclusion

In conclusion, Color models and color image processing enhance the understanding and manipulation of images in different applications. Optimization of most specific image processing tasks could be achieved by using different color spaces like RGB, CMY, HSV, or YCbCr. Efficient image analysis and visualization occur when different techniques such as filtering, segmentation, or transformation are brought to the scene. These methods achieve the desired goal in computer vision, medical imaging, and multimedia. Ultimately, the use of color models contributes to image processing tasks by ensuring accuracy and flexibility in their handling.