FINAL YEAR PROJECT

IOT-BASED AIR QUALITY INDEX METER SYSTEM

GROUP NUMBER:G06

NAME:

HUZAIFA SHAHBAZ

AQSA SHAH

REGISTRATION NUMBER:

21-CE-009

21-CE-039

DATE:

26thMay,2025

Department of Computer Engineering,

HITEC University, Taxila

Table of Contents

1.	MATLAB CODES	3

1.1	AQITABLEINDEX	3

1.2	AVERAGING FILTER	4

1.3	KALMAN FILTER	5

1.4	MICROCONTROLLERS	6

1.5	MQ GAS SENSORS	8

1.6	PM SENSORS	10

1.7	WEATHERPROOF HOUSING	11

1.5	CODE OF PYTHON	12

2.1	ALERTS AND NOTIFICATIONS	12

2.2	AVERAGING FILTER	13

2.3	KALMAN FILTER	15

2.4	SMS ALERT WITH AQI ALERTS	16

3.	ARDUINO FRAMEWORK+VS FIRMWARE CODING	17

3.1	MQ-5 WITH ARDUINO FRAMEWORK AND VS FIRMWARE CODING	17

3.2	MQ-135 WITH ARDUINO FRAMEWORK AND VS FIRMWARE CODING	19

3.3	PM2.5 AND PM 10 SENSORS WITH ARDUINO FRAMEWORK AND VS CODE FIRMWARE CODING	21

3.4	WITH ARDUINO FRAMEWORK AND VS CODE FIRMWARE CODING	22

2.5	FINAL VERSION OF ARDUINO UNO WITH VS CODE FRAMEWORK	25

4.	BLYNK APP	27

4.1	BLYNK APP WITH ESP8266 IN VS CODE FIRMWARE CODING	27

5.	ANDROID STUDIO	36

5.1	MAIN ACTIVITY JAVA CODE OF AIR QUALITY INDEX METER SYSTEM	36

5.1.1	XML LINAER LAYOUT OF AIR QUALITY INDEX METER SYSTEM	42

5.2	AIR QUALITY ACTIVITY JAVA CODE	45

CHAPTER ONE

# MATLAB CODES

# AQITABLEINDEX

THE MATLAB CODE OF AIR QUALITY TABLE INDEX AS SHOWN BELOW:

% MATLAB code to define AQI ranges and plot the graph

% Define AQI categories

AQI_Categories = {"Good", "Moderate", "Unhealthy for Sensitive Groups", "Unhealthy", "Very Unhealthy", "Hazardous"};

AQI_Ranges = [0 50; 51 100; 101 150; 151 200; 201 300; 301 500];

Health_Impacts = {"No threat", "Possibly concerning for sensitive individuals", ...

"Unhealthy for sensitive groups", "Unhealthy for everyone", ...

"Entire population likely to be affected", "Serious health effects"};

% Define colors corresponding to each AQI range

Colors = [0 1 0;     % Green

1 1 0;     % Yellow

1 0.5 0;   % Orange

1 0 0;     % Red

0.5 0 0.5; % Purple

0.3 0 0.3];% Dark Purple

% Create a figure for the bar plot

figure;

hold on;

% Loop through AQI categories and plot bars

for i = 1:length(AQI_Categories)

bar(i, AQI_Ranges(i, 2), 'FaceColor', Colors(i, :), 'EdgeColor', 'none');

end

% Customize the plot

set(gca, 'XTick', 1:length(AQI_Categories), 'XTickLabel', AQI_Categories);

xlabel('AQI Categories');

ylabel('AQI Range');

title('Air Quality Index (AQI) Categories');

grid on;

ylim([0 550]);

legend(AQI_Categories, 'Location', 'northwest');

% Display AQI table in the command window

disp('AQI Table:');

disp('----------------------------------------------');

for i = 1:length(AQI_Categories)

fprintf('Category: %-30s | Range: %3d-%3d | Health Impact: %s\n', AQI_Categories{i}, AQI_Ranges(i, 1), AQI_Ranges(i, 2), Health_Impacts{i});

end

hold off;

# AVERAGING FILTER

THE MATLAB CODE OF AVERAGING FILTER AS SHOWN BELOW:

% Averaging Filter for Air Quality Sensors

% MQ-5, MQ-135, PM2.5, PM10

clc;

clear;

close all;

% Simulated sensor data (replace with actual readings)

numSamples = 100; % number of data points

windowSize = 5;   % moving average window size

% Simulated noisy data (you can replace this with real-time data input)

mq5_raw   = 100 + 10*randn(1, numSamples);   % MQ-5: gas detection

mq135_raw = 200 + 15*randn(1, numSamples);   % MQ-135: air quality

pm25_raw  = 35  + 5*randn(1, numSamples);    % PM2.5 concentration

pm10_raw  = 50  + 7*randn(1, numSamples);    % PM10 concentration

% Averaging filter using built-in 'movmean'

mq5_filtered   = movmean(mq5_raw, windowSize);

mq135_filtered = movmean(mq135_raw, windowSize);

pm25_filtered  = movmean(pm25_raw, windowSize);

pm10_filtered  = movmean(pm10_raw, windowSize);

% Plot the results

figure;

subplot(2,2,1);

plot(mq5_raw, '--r'); hold on; plot(mq5_filtered, 'b', 'LineWidth', 1.5);

title('MQ-5 Sensor'); legend('Raw','Filtered'); grid on;

subplot(2,2,2);

plot(mq135_raw, '--g'); hold on; plot(mq135_filtered, 'b', 'LineWidth', 1.5);

title('MQ-135 Sensor'); legend('Raw','Filtered'); grid on;

subplot(2,2,3);

plot(pm25_raw, '--m'); hold on; plot(pm25_filtered, 'b', 'LineWidth', 1.5);

title('PM2.5 Sensor'); legend('Raw','Filtered'); grid on;

subplot(2,2,4);

plot(pm10_raw, '--k'); hold on; plot(pm10_filtered, 'b', 'LineWidth', 1.5);

title('PM10 Sensor'); legend('Raw','Filtered'); grid on;

sgtitle('Averaging Filter on Sensor Readings');

# KALMAN FILTER

THE MATLAB CODE OF KALMAN FILTER AS SHOWN BELOW:

clc;

clear;

% --- Simulation Parameters ---

num_samples = 100; % Number of sensor readings

% Simulate raw noisy sensor data (Replace this with real data input)

mq5_raw    = 200 + 10*randn(1, num_samples);  % Example values in ppm

mq135_raw  = 300 + 15*randn(1, num_samples);  % Example values in ppm

pm25_raw   = 50 + 5*randn(1, num_samples);    % Example values in µg/m³

pm10_raw   = 80 + 7*randn(1, num_samples);    % Example values in µg/m³

% Initialize Kalman filter variables for each sensor

% Initial state estimate and error covariance

x = [0 0 0 0]; % initial state estimates

P = [1 1 1 1]; % initial estimation error covariance

% Define process noise covariance (Q) and measurement noise covariance (R)

Q = [0.01 0.01 0.01 0.01];  % process noise

R = [25 30 10 20];          % measurement noise

% Store filtered output

mq5_filtered = zeros(1, num_samples);

mq135_filtered = zeros(1, num_samples);

pm25_filtered = zeros(1, num_samples);

pm10_filtered = zeros(1, num_samples);

% Kalman Filter Loop

for k = 1:num_samples

% 1. Prediction Step

x_pred = x;

P_pred = P + Q;

% 2. Update Step with new measurement

z = [mq5_raw(k), mq135_raw(k), pm25_raw(k), pm10_raw(k)];

K = P_pred ./ (P_pred + R);              % Kalman Gain

x = x_pred + K .* (z - x_pred);          % Updated estimate

P = (1 - K) .* P_pred;                   % Updated error covariance

% Save filtered results

mq5_filtered(k) = x(1);

mq135_filtered(k) = x(2);

pm25_filtered(k) = x(3);

pm10_filtered(k) = x(4);

end

% --- Plotting Results ---

figure;

subplot(2,2,1);

plot(mq5_raw, 'r--'); hold on;

plot(mq5_filtered, 'b-');

title('MQ-5 Sensor'); legend('Raw','Filtered');

subplot(2,2,2);

plot(mq135_raw, 'r--'); hold on;

plot(mq135_filtered, 'b-');

title('MQ-135 Sensor'); legend('Raw','Filtered');

subplot(2,2,3);

plot(pm25_raw, 'r--'); hold on;

plot(pm25_filtered, 'b-');

title('PM2.5 Sensor'); legend('Raw','Filtered');

subplot(2,2,4);

plot(pm10_raw, 'r--'); hold on;

plot(pm10_filtered, 'b-');

title('PM10 Sensor'); legend('Raw','Filtered');

sgtitle('Kalman Filter on Sensor Readings');

# MICROCONTROLLERS

THE MATLAB CODE OF MICROCONTROLLERS AS SHOWN BELOW:

% Define microcontroller specifications

microcontrollers = {'ESP32', 'ESP8266', 'Arduino UNO'};

% CPU Speed in MHz

cpu_speed = [240, 160, 16];

% RAM in KB

ram = [520, 160, 2];

% Flash Memory in MB

flash = [4, 4, 0.032];

% GPIO Pins

gpio = [36, 17, 14];

% ADC Resolution (bits)

adc = [12, 10, 10];

% Create figure with subplots

figure('Name', 'Microcontroller Comparison', 'Position', [100, 100, 1200, 800]);

% 1. CPU Speed Comparison

subplot(2,3,1)

bar(cpu_speed)

title('CPU Speed Comparison')

xlabel('Microcontrollers')

ylabel('Speed (MHz)')

set(gca, 'XTick', 1:3, 'XTickLabel', microcontrollers)

grid on

% 2. RAM Comparison

subplot(2,3,2)

bar(ram)

title('RAM Comparison')

xlabel('Microcontrollers')

ylabel('RAM (KB)')

set(gca, 'XTick', 1:3, 'XTickLabel', microcontrollers)

grid on

% 3. Flash Memory Comparison

subplot(2,3,3)

bar(flash)

title('Flash Memory Comparison')

xlabel('Microcontrollers')

ylabel('Flash (MB)')

set(gca, 'XTick', 1:3, 'XTickLabel', microcontrollers)

grid on

% 4. GPIO Pins Comparison

subplot(2,3,4)

bar(gpio)

title('GPIO Pins Comparison')

xlabel('Microcontrollers')

ylabel('Number of GPIO Pins')

set(gca, 'XTick', 1:3, 'XTickLabel', microcontrollers)

grid on

% 5. ADC Resolution Comparison

subplot(2,3,5)

bar(adc)

title('ADC Resolution Comparison')

xlabel('Microcontrollers')

ylabel('Resolution (bits)')

set(gca, 'XTick', 1:3, 'XTickLabel', microcontrollers)

grid on

% 6. Radar Chart for Overall Comparison

subplot(2,3,6)

% Normalize the data for radar chart

normalized_data = [

cpu_speed./max(cpu_speed);

ram./max(ram);

flash./max(flash);

gpio./max(gpio);

adc./max(adc)

];

categories = {'CPU Speed', 'RAM', 'Flash', 'GPIO', 'ADC'};

spider_plot(normalized_data', categories, 'Microcontrollers', microcontrollers);

title('Overall Comparison (Normalized)')

% Add color to all plots

colormap('summer')

% Adjust layout

sgtitle('Microcontroller Specifications Comparison')

set(gcf, 'Color', 'white')

% Function for creating spider plot

function spider_plot(P, categories, legendTitle, legendLabels)

% Number of variables

num_vars = size(P, 2);

% Angles for each axis

angles = linspace(0, 2*pi, num_vars+1);

% Plot data points

colors = lines(size(P, 1));

% Create the plot

polarplot([angles(1:end-1), angles(1)], [P(1,:), P(1,1)], '-o', 'LineWidth', 2);

hold on

for i = 2:size(P, 1)

polarplot([angles(1:end-1), angles(1)], [P(i,:), P(i,1)], '-o', 'LineWidth', 2);

end

% Fill in the plot

pax = gca;

pax.ThetaDir = 'clockwise';

pax.ThetaZeroLocation = 'top';

% Set the labels

pax.ThetaTick = rad2deg(angles(1:end-1));

pax.ThetaTickLabel = categories;

% Add legend

legend(legendLabels, 'Location', 'southoutside')

grid on

end

# MQ GAS SENSORS

THE MATLAB CODE OF MQ GAS SENSORS(MQ-5 AND MQ-135) AS SHOWN BELOW

% MQ-5 and MQ-135 Gas Sensors Data and Plotting

clc;

clear;

close all;

% Constants (example calibration values from datasheets)

RL = 10; % Load resistance in kOhms

Vcc = 5; % Supply voltage in volts

% MQ-5 Sensor (SnO2, H2, LPG, CH4, CO, Alcohol)

MQ5_Ro = 10; % Sensor resistance in clean air (kOhms)

MQ5_Gases = {'SnO2', 'H2', 'LPG', 'CH4', 'CO', 'Alcohol'};

MQ5_Rs_Ro = [

1.2, 1.1, 1.0, 0.9, 0.8, 0.7; % Rs/Ro for SnO2

1.1, 0.9, 0.7, 0.6, 0.5, 0.4; % Rs/Ro for H2

1.0, 0.8, 0.6, 0.5, 0.4, 0.3; % Rs/Ro for LPG

0.9, 0.7, 0.5, 0.4, 0.3, 0.2; % Rs/Ro for CH4

0.8, 0.6, 0.4, 0.3, 0.2, 0.1; % Rs/Ro for CO

0.7, 0.5, 0.3, 0.2, 0.1, 0.05 % Rs/Ro for Alcohol

];

MQ5_PPM = [10, 50, 100, 200, 300, 400]; % Concentrations in ppm

% MQ-135 Sensor (SO2, NO2, O3, CO)

MQ135_Ro = 10; % Sensor resistance in clean air (kOhms)

MQ135_Gases = {'SO2', 'NO2', 'O3', 'CO'};

MQ135_Rs_Ro = [

1.0, 0.9, 0.8, 0.7; % Rs/Ro for SO2

0.9, 0.8, 0.7, 0.6; % Rs/Ro for NO2

0.8, 0.7, 0.6, 0.5; % Rs/Ro for O3

0.7, 0.6, 0.5, 0.4  % Rs/Ro for CO

];

MQ135_PPM = [20, 60, 120, 240]; % Concentrations in ppm

% Calculate Rs and voltage output for MQ-5

MQ5_Rs = MQ5_Ro * MQ5_Rs_Ro; % Sensor resistance for MQ-5

MQ5_Vout = MQ5_Rs ./ (MQ5_Rs + RL) * Vcc; % Voltage output for MQ-5

% Calculate Rs and voltage output for MQ-135

MQ135_Rs = MQ135_Ro * MQ135_Rs_Ro; % Sensor resistance for MQ-135

MQ135_Vout = MQ135_Rs ./ (MQ135_Rs + RL) * Vcc; % Voltage output for MQ-135

% Plotting MQ-5 Sensor Data

figure;

subplot(2, 1, 1);

hold on;

for i = 1:length(MQ5_Gases)

plot(MQ5_PPM, MQ5_Vout(i, :), '-o', 'LineWidth', 1.5, 'DisplayName', MQ5_Gases{i});

end

title('MQ-5 Sensor (SnO2, H2, LPG, CH4, CO, Alcohol)');

xlabel('Gas Concentration (ppm)');

ylabel('Voltage Output (V)');

grid on;

legend('show');

hold off;

% Plotting MQ-135 Sensor Data

subplot(2, 1, 2);

hold on;

for i = 1:length(MQ135_Gases)

plot(MQ135_PPM, MQ135_Vout(i, :), '-o', 'LineWidth', 1.5, 'DisplayName', MQ135_Gases{i});

end

title('MQ-135 Sensor (SO2, NO2, O3, CO)');

xlabel('Gas Concentration (ppm)');

ylabel('Voltage Output (V)');

grid on;

legend('show');

hold off;

# PM SENSORS

THE MATLAB CODE OF PM SENSORS(PM 2.5 AND PM 10) AS SHOWN BELOW

% MATLAB Code for Simulating and Plotting PM2.5 and PM10 Data

% Clear workspace and command window

clear;

clc;

% Simulate time (in seconds)

simulationTime = 0:1:100; % 100 seconds at 1-second intervals

% Simulate PM2.5 and PM10 sensor data (randomly generated for this example)

% Replace this with real sensor data if available

PM2_5 = 10 + 5*randn(1, length(simulationTime)); % Mean 10, std deviation 5

PM10 = 20 + 8*randn(1, length(simulationTime));  % Mean 20, std deviation 8

% Ensure no negative values in simulated data

PM2_5(PM2_5 < 0) = 0;

PM10(PM10 < 0) = 0;

% Plot the data

figure;

plot(simulationTime, PM2_5, '-o', 'LineWidth', 1.5, 'MarkerSize', 6, 'Color', [0, 0.4470, 0.7410]); % PM2.5 plot

hold on;

plot(simulationTime, PM10, '-s', 'LineWidth', 1.5, 'MarkerSize', 6, 'Color', [0.8500, 0.3250, 0.0980]); % PM10 plot

hold off;

% Add title and labels

title('PM2.5 and PM10 Concentration Over Time', 'FontSize', 14);

xlabel('Time (seconds)', 'FontSize', 12);

ylabel('Concentration (\mug/m^3)', 'FontSize', 12);

legend({'PM2.5', 'PM10'}, 'FontSize', 12, 'Location', 'northwest');

% Add grid for better visualization

grid on;

% Highlight thresholds (if required)

hold on;

yline(35, '--', 'PM2.5 Threshold', 'Color', [0, 0.4470, 0.7410], 'LineWidth', 1);

yline(50, '--', 'PM10 Threshold', 'Color', [0.8500, 0.3250, 0.0980], 'LineWidth', 1);

hold off;

% Display summary statistics

disp('Summary of PM2.5 and PM10 Data:');

disp(['Mean PM2.5: ', num2str(mean(PM2_5)), ' \mug/m^3']);

disp(['Mean PM10: ', num2str(mean(PM10)), ' \mug/m^3']);

disp(['Max PM2.5: ', num2str(max(PM2_5)), ' \mug/m^3']);

disp(['Max PM10: ', num2str(max(PM10)), ' \mug/m^3']);

disp(['Min PM2.5: ', num2str(min(PM2_5)), ' \mug/m^3']);

disp(['Min PM10: ', num2str(min(PM10)), ' \mug/m^3']);

# WEATHERPROOF HOUSING

THE MATLAB CODE OF WEATHERPROOF HOUSING AS SHOWN BELOW

% Weatherproof Housing Simulation of AQI Meter System

% Description: Simulate internal environment inside AQI enclosure

% Time setup

t = 0:60:24*3600; % simulate for 24 hours, 1-minute step

n = length(t);

% Ambient Environment (example data or from sensor logs)

ambient_temp = 35 + 5*sin(2*pi*t/(24*3600));    % ambient temp in °C

ambient_humidity = 50 + 20*sin(2*pi*t/(12*3600)); % RH in %

% Housing properties

thermal_resistance = 0.1;  % °C/W

thermal_capacitance = 500; % J/°C

initial_internal_temp = 30;

humidity_damping = 0.05; % how fast internal RH equalizes

initial_internal_humidity = 45;

% Initialize

internal_temp = zeros(1, n);

internal_temp(1) = initial_internal_temp;

internal_humidity = zeros(1, n);

internal_humidity(1) = initial_internal_humidity;

% Heat and moisture transfer simulation

for i = 2:n

dt = t(i) - t(i-1);

% Temperature modeling (RC circuit analogy)

dT = (ambient_temp(i-1) - internal_temp(i-1)) / thermal_resistance;

internal_temp(i) = internal_temp(i-1) + dt * (dT / thermal_capacitance);

% Humidity modeling (simple exponential filter)

internal_humidity(i) = internal_humidity(i-1) + ...

humidity_damping * (ambient_humidity(i-1) - internal_humidity(i-1));

end

% Plot results

figure;

subplot(2,1,1);

plot(t/3600, ambient_temp, 'r--', t/3600, internal_temp, 'b');

xlabel('Time (hours)');

ylabel('Temperature (°C)');

legend('Ambient', 'Inside Housing');

title('Temperature Profile');

subplot(2,1,2);

plot(t/3600, ambient_humidity, 'g--', t/3600, internal_humidity, 'b');

xlabel('Time (hours)');

ylabel('Humidity (%)');

legend('Ambient', 'Inside Housing');

title('Humidity Profile');

CHAPTER TWO

# CODE OF PYTHON

# ALERTS AND NOTIFICATIONS

THE PYTHON CODE OF ALERTS AND NOTIFICATIONS

import smtplib

from email.mime.text import MIMEText

from datetime import datetime

# ------------------ AQI Thresholds (Example Thresholds in ppm or µg/m³) ------------------

AQI_THRESHOLDS = {

"PM2.5": 100,    # µg/m³

"PM10": 150,     # µg/m³

"CO": 10,        # ppm

"SO2": 13.4,     # ppm (corrected key name)

"MQ-5": 1000,    # ppm (LPG, CH4, CO – define limit based on calibration)

"MQ-135": 300    # ppm (CO2, NH3, Benzene – example threshold)

}

# ------------------ Email Notification Setup ------------------

EMAIL_SENDER = "airquality786@gmail.com"

EMAIL_RECEIVER = "airqualityindexmeter09039@example.com"

SMTP_SERVER = "smtp.gmail.com"

SMTP_PORT = 587

EMAIL_PASSWORD = "21CE009039"  # Use Gmail app password, not your email password

def send_email(subject, message):

msg = MIMEText(message)

msg["Subject"] = subject

msg["From"] = EMAIL_SENDER

msg["To"] = EMAIL_RECEIVER

try:

with smtplib.SMTP(SMTP_SERVER, SMTP_PORT) as server:

server.starttls()

server.login(EMAIL_SENDER, EMAIL_PASSWORD)

server.send_message(msg)

print("✅ Email alert sent successfully.")

except Exception as e:

print(f"❌ Error sending email: {e}")

# ------------------ Alert Function ------------------

def check_aqi_and_alert(data):

alerts = []

for key, value in data.items():

if key in AQI_THRESHOLDS and value > AQI_THRESHOLDS[key]:

alerts.append(f"⚠️ {key} Level High: {value} (Threshold: {AQI_THRESHOLDS[key]})")

if alerts:

timestamp = datetime.now().strftime("%Y-%m-%d %H:%M:%S")

alert_message = "\n".join(alerts)

full_message = f"🕒 Alert Time: {timestamp}\n\n{alert_message}"

print(full_message)

send_email("🚨 Air Quality Alert!", full_message)

# ------------------ Simulated Data Example ------------------

if __name__ == "__main__":

# Example sensor readings (replace with real-time sensor data)

sensor_data = {

"PM2.5": 120,

"PM10": 180,

"CO": 12.5,

"SO2": 14.0,

"MQ-5": 1500,

"MQ-135": 350

}

check_aqi_and_alert(sensor_data)

# AVERAGING FILTER

THE PYTHON CODE OF AVERAGING FILTER AS SHOWN BELOW:

from collections import deque

import time

import random  # Replace with actual sensor library (e.g., from Adafruit_MQ135 import MQ135)

class AveragingFilter:

def __init__(self, window_size=10):

self.window_size = window_size

self.pm25_values = deque(maxlen=window_size)

self.pm10_values = deque(maxlen=window_size)

self.mq135_values = deque(maxlen=window_size)

def update(self, pm25, pm10, mq135):

self.pm25_values.append(pm25)

self.pm10_values.append(pm10)

self.mq135_values.append(mq135)

def get_averages(self):

pm25_avg = sum(self.pm25_values) / len(self.pm25_values) if self.pm25_values else 0

pm10_avg = sum(self.pm10_values) / len(self.pm10_values) if self.pm10_values else 0

mq135_avg = sum(self.mq135_values) / len(self.mq135_values) if self.mq135_values else 0

return pm25_avg, pm10_avg, mq135_avg

# Example use case (simulate real-time readings)

if __name__ == "__main__":

filter = AveragingFilter(window_size=5)

for i in range(20):  # Simulating 20 sensor readings

# Replace these with actual sensor values

pm25 = random.uniform(10, 100)

pm10 = random.uniform(20, 150)

mq135 = random.uniform(200, 400)

filter.update(pm25, pm10, mq135)

avg_pm25, avg_pm10, avg_mq135 = filter.get_averages()

print(f"Filtered PM2.5: {avg_pm25:.2f}, PM10: {avg_pm10:.2f}, MQ135: {avg_mq135:.2f}")

time.sleep(1)  # Simulate time gap between readings

# KALMAN FILTER

THE PYTHON CODE OF KALMAN FILTER AS SHOWN BELOW:

class KalmanFilter:

def __init__(self, process_variance, measurement_variance, estimated_error=1.0, initial_value=0.0):

"""

process_variance: Q - Process noise (small value like 1e-5)

measurement_variance: R - Sensor noise (depends on your sensor)

estimated_error: P - Initial estimation error

initial_value: x - Initial estimated value

"""

self.Q = process_variance

self.R = measurement_variance

self.P = estimated_error

self.x = initial_value

self.K = 0.0  # Kalman gain

def update(self, measurement):

# Prediction Update

self.P = self.P + self.Q

# Measurement Update

self.K = self.P / (self.P + self.R)

self.x = self.x + self.K * (measurement - self.x)

self.P = (1 - self.K) * self.P

return self.x

# SMS ALERT WITH AQI ALERTS

THE PYTHON CODE OF SMS ALERT AS SHOWN BELOW:

from twilio.rest import Client

# Twilio credentials from your account

account_sid = 'G3ozx5xtEqGAGi3VdsQGKGHxuwqSJDTn38vEUEREQweQ'

auth_token = '0364c7abb1e33834e1f7a6c919388ca8'

# Initialize Twilio client

client = Client(account_sid, auth_token)

# Simulated sensor data

aqi_value = 175  # Example AQI value

# Define message severity

if aqi_value <= 50:

status = "Good"

elif aqi_value <= 100:

status = "Moderate"

elif aqi_value <= 150:

status = "Unhealthy for Sensitive Groups"

elif aqi_value <= 200:

status = "Unhealthy"

elif aqi_value <= 300:

status = "Very Unhealthy"

else:

status = "Hazardous"

# Create the alert message

message_body = f"AQI Alert! Current AQI: {aqi_value} ({status}). Take precautions if needed."

# Send SMS

message = client.messages.create(

body=message_body,

from_='+1234567890',  # Your Twilio phone number

to='+9187654321'      # Your verified recipient number

)

print("Message SID:", message.sid)

CHAPTER THREE

# ARDUINO FRAMEWORK+VS FIRMWARE CODING

# MQ-5 WITH ARDUINO FRAMEWORK AND VS FIRMWARE CODING

THE MQ-5 GAS SENSORS  WITH ARDUINO UNO WITH ARDINO FRAMEWORK AS SHOWN BELOW AS:

// MQ-5 Gas Sensor with Arduino Uno

// For AQI estimation (LPG, CH4, CO, H2 detection)

// Author: HuzaifaShahbaz | May 2025

#define MQ5_ANALOG A0    // MQ-5 analog output pin

#define RL_VALUE 10.0    // Load resistance in kilo ohms

#define RO_CLEAN_AIR_FACTOR 6.5 // Sensor resistance in clean air (from datasheet)

// Calibration and calculation variables

float Ro = 10.0;         // Initial Ro value (will be calibrated)

float LPGCurve[3] = {2.3, 0.21, -0.47}; // LPG curve

float CH4Curve[3] = {2.3, 0.72, -0.34}; // Methane curve

void setup() {

Serial.begin(9600);

Serial.println("Calibrating MQ-5 Sensor...");

Ro = MQCalibration(MQ5_ANALOG);  // Calibrate in clean air

Serial.print("Calibration done. Ro = ");

Serial.println(Ro);

}

void loop() {

float rs = MQResistanceCalculation(analogRead(MQ5_ANALOG));

float ratio = rs / Ro;

float ppmLPG = pow(10, ((log10(ratio) - LPGCurve[1]) / LPGCurve[2]));

float ppmCH4 = pow(10, ((log10(ratio) - CH4Curve[1]) / CH4Curve[2]));

Serial.print("LPG Concentration: ");

Serial.print(ppmLPG);

Serial.print(" ppm\t");

Serial.print("CH4 Concentration: ");

Serial.print(ppmCH4);

Serial.println(" ppm");

delay(2000);

}

// Function to calculate sensor resistance

float MQResistanceCalculation(int raw_adc) {

return (((float)RL_VALUE * (1023 - raw_adc)) / raw_adc);

}

// Function to calibrate sensor in clean air

float MQCalibration(int mq_pin) {

int i;

float val = 0;

for (i = 0; i < 50; i++) {

val += MQResistanceCalculation(analogRead(mq_pin));

delay(500);

}

val = val / 50;

return val / RO_CLEAN_AIR_FACTOR;

}

# MQ-135 WITH ARDUINO FRAMEWORK AND VS FIRMWARE CODING

THE MQ-135 GAS SENSORS  WITH ARDUINO UNO WITH ARDINO FRAMEWORK AS SHOWN BELOW AS:

#define MQ_135 A0//ANALOG PIN CONNECTED TO MQ-135

float air quality; //VARIABLE TO HOLD FINAL AIR QUALITY LEVEL

void setup()

{

Serial.begin(9600);

pinMode(MQ_135PIN,INPUT);

}

void loop()

{

int mq135_value=analogRead(MQ135_PIN);

//MAP THE RAW SENSOR VALUE TO AQI SCALE APPROXIMATION

air_quality=map(mq135_value,0,1023,0,500);//0-500:APPROX AQI RANGE

Serial.print("Raw Sensor Value:")

Serial.print(mq135_value);

Serial.print("=>Approx AQI:")

Serial.print(air_quality);

//AQI CATEGORIZATION

if(air_quality<=50)

{

Serial.print("Air quality:Good");

}

else if(air_quality<=100)

{

Serial.print("Air quality:Moderate")

}

else if(air_quality<=150)

{

Serial.print("Air quality:Unhealthy for sensitive Groups");

}

else if(air_quality<=200)

{

Serial.print("Air quality:Unhealthy");

}

else if(air_quality<=300)

{

Serial.print("Air quality:Very Unhealthy")

}

else

{

Serial.println("Air Quality:Hazardous")

}

}

# PM2.5 AND PM 10 SENSORS WITH ARDUINO FRAMEWORK AND VS CODE FIRMWARE CODING

THE PM2.5 AND PM10 SENSORS  WITH ARDUINO UNO WITH ARDINO FRAMEWORK AS SHOWN BELOW AS:

#include<Software Serial.h>

#include<PMS.h>

SoftwareSerial pmSerial(10,11);//RX,TX

PMS pms(pmsSerial);

PMS::DATA data;

void setup()

{

Serial.begin(9600);

pmsSerial.begin(9600);

pms.wakeUp();//wake sensor if sleeping

delay(1000);

Serial.println("Air quality index System Started...");

}

void loop()

{

if(pms.readUntil(data))

{

Serial.print("PM 1.0 (ug/m3): "); Serial.println(data.PM_AE_UG_1_0);

Serial.print("PM 2.5 (ug/m3): "); Serial.println(data.PM_AE_UG_2_5);

Serial.print("PM 10.0 (ug/m3): "); Serial.println(data.PM_AE_UG_10_0);

//Calculate basic AQI (OPTIONAL ENHANCEMENTS)

if(data.PM_AE_UG_2_5<=12)

{

Serial.println("AQI:Good");

}

else if(data.PM_AE_UG_2_5<=35.4)

{

Serial.println("AQI:Moderate");

}

else

{

Serial.println("AQI:Unhealthy");

}

}

delay(5000);//DELAY BETWEEN  READINGS

}

# WITH ARDUINO FRAMEWORK AND VS CODE FIRMWARE CODING

THE HC-05 BLUETOOTH MODULE  WITH ARDUINO UNO WITH ARDINO FRAMEWORK AS SHOWN BELOW AS:

#include <SoftwareSerial.h>

// For Bluetooth

SoftwareSerial BTSerial(10, 11); // RX | TX (HC-05 TX to Pin 10, RX to Pin 11)

// Sensor Pins

#define MQ5_PIN A0

#define MQ135_PIN A1

// PMS5003 UART

SoftwareSerial PMS_Serial(2, 3); // PMS TX to pin 2, RX to pin 3

uint8_t pmsData[32];

// AQI Calculation placeholders (simplified)

int getMQ5PPM()

{

int analogValue = analogRead(MQ5_PIN);

return map(analogValue, 0, 1023, 0, 1000);  // Convert to PPM approx

}

int getMQ135PPM()

{

int analogValue = analogRead(MQ135_PIN);

return map(analogValue, 0, 1023, 0, 1000);  // Convert to PPM approx

}

// Reads PM2.5 and PM10 values from PMS5003

bool readPMSData(uint16_t &pm25, uint16_t &pm10)

{

if (PMS_Serial.available() >= 32)

{

if (PMS_Serial.read() == 0x42)

{

if (PMS_Serial.read() == 0x4d)

{

for (int i = 0; i < 30; i++)

{

pmsData[i] = PMS_Serial.read();

}

pm25 = (pmsData[6] << 8) + pmsData[7];  // PM2.5 (standard particles)

pm10 = (pmsData[8] << 8) + pmsData[9];  // PM10

return true;

}

}

}

return false;

}

void setup()

{

Serial.begin(9600);

BTSerial.begin(9600);

PMS_Serial.begin(9600);

pinMode(MQ5_PIN, INPUT);

pinMode(MQ135_PIN, INPUT);

Serial.println("AQI Meter System Initialized...");

BTSerial.println("Bluetooth AQI Meter Initialized");

}

void loop()

{

int mq5 = getMQ5PPM();

int mq135 = getMQ135PPM();

uint16_t pm25 = 0, pm10 = 0;

bool pmDataReady = readPMSData(pm25, pm10);

String data = "MQ-5: " + String(mq5) + " ppm | MQ-135: " + String(mq135) + " ppm";

if (pmDataReady)

{

data += " | PM2.5: " + String(pm25) + " µg/m3 | PM10: " + String(pm10) + " µg/m3";

} else

{

data += " | PM Sensor: Not Ready";

}

Serial.println(data);

BTSerial.println(data);  // Send data via Bluetooth

delay(2000); // Delay between readings

}

# FINAL VERSION OF ARDUINO UNO WITH VS CODE FRAMEWORK

The final version code of Arduino uno with VS code framework with PlatformIO ide as shown below:

#include <Arduino.h>

#include <SoftwareSerial.h>

#include <Adafruit_PM25AQI.h>

#include <Wire.h>

#define MQ135_PIN A0

#define MQ5_PIN   A1

// HC-05 Bluetooth connected to D10 (TX), D11 (RX)

SoftwareSerial bluetooth(10, 11); // RX, TX

// PM2.5/PM10 UART sensor on Serial or SoftwareSerial

Adafruit_PM25AQI aqi = Adafruit_PM25AQI();

void setup() {

Serial.begin(9600);           // Monitor serial

bluetooth.begin(9600);        // HC-05 Bluetooth module

delay(1000);

Serial.println("AQI Meter Starting...");

bluetooth.println("Bluetooth AQI Meter Ready");

if (!aqi.begin_UART(&Serial1)) { // Replace Serial1 with SoftwareSerial if needed

Serial.println("Could not find PM2.5 sensor!");

bluetooth.println("PM2.5 sensor error");

while (1) delay(10);

}

}

void loop() {

// Read analog values

int mq135_raw = analogRead(MQ135_PIN);

int mq5_raw = analogRead(MQ5_PIN);

// Simple conversion (mock)

float mq135_ppm = mq135_raw * (5.0 / 1023.0) * 100;  // Scaled

float mq5_ppm   = mq5_raw   * (5.0 / 1023.0) * 100;

// Read PM2.5 / PM10

PM25_AQI_Data data;

if (aqi.read(&data)) {

Serial.println("----- AQI Data -----");

Serial.print("PM 2.5: "); Serial.println(data.pm25_standard);

Serial.print("PM 10 : "); Serial.println(data.pm10_standard);

Serial.print("MQ135 (ppm): "); Serial.println(mq135_ppm);

Serial.print("MQ5   (ppm): "); Serial.println(mq5_ppm);

// Send over Bluetooth

bluetooth.print("PM2.5: "); bluetooth.print(data.pm25_standard); bluetooth.println(" µg/m3");

bluetooth.print("PM10 : "); bluetooth.print(data.pm10_standard); bluetooth.println(" µg/m3");

bluetooth.print("MQ135: "); bluetooth.print(mq135_ppm); bluetooth.println(" ppm");

bluetooth.print("MQ5  : "); bluetooth.print(mq5_ppm); bluetooth.println(" ppm");

}

delay(2000);

}

CHAPTER FOUR

# BLYNK APP

# BLYNK APP WITH ESP8266 IN VS CODE FIRMWARE CODING

The blynk app code by using ESP8266 IN VS CODE FIRMWARE CODING:

#include <Arduino.h>

#include<BlynkSimpleEsp8266.h>

#include<SoftwareSerial.h>

#include<PMS.h>

//Blynk Authentication token

#define BLYNK_AUTH_TOKEN "aCW4BGaSVWaLGuM6qncZy1qQpoXD_6nj"

#define BLYNK_PRINT Serial

//WiFi credentials

const char* ssid = "HuzaifaandAqsaNet";

const char*password="21CE009039";

// Pin definitions

#define MQ5_PIN A0       // MQ-5 sensor connected to analog pin A0

#define MQ135_PIN D0     // MQ-135 sensor connected to digital pin D0

#define PM_TX_PIN D7     // PMS5003 TX pin connected to D7

#define PM_RX_PIN D8     // PMS5003 RX pin connected to D8

// Virtual pins for Blynk

#define VPIN_MQ5 V0      // Virtual pin for MQ-5 readings

#define VPIN_MQ135 V1    // Virtual pin for MQ-135 readings

#define VPIN_PM25 V2     // Virtual pin for PM2.5 readings

#define VPIN_PM10 V3     // Virtual pin for PM10 readings

#define VPIN_AIR_QUALITY V4  // Virtual pin for overall air quality index

// Constants for sensors

const float MQ5_THRESHOLD = 400.0;   // Threshold for MQ-5 danger level

const float MQ135_THRESHOLD = 600.0; // Threshold for MQ-135 danger level

const float PM25_THRESHOLD = 35.0;   // Threshold for PM2.5 danger level (μg/m³)

const float PM10_THRESHOLD = 50.0;   // Threshold for PM10 danger level (μg/m³)

// Create software serial for PMS5003 sensor

SoftwareSerial pmSerial(PM_RX_PIN, PM_TX_PIN); // RX, TX

PMS pms(pmSerial);                             // Create PMS sensor instance

PMS::DATA pmsData;

// Variables to store sensor readings

float mq5Value = 0;

float mq135Value = 0;

float pm25Value = 0;

float pm10Value = 0;

int airQualityIndex = 0;  // 0-100 scale

// Timer variables

unsigned long previousMillis = 0;

const long interval = 2000;  // Reading interval in milliseconds

// Function prototypes

void readMQ5Sensor();

void readMQ135Sensor();

void readPMSensor();

void calculateAirQualityIndex();

void sendDataToBlynk();

/**

* Setup function - runs once at startup

*/

void setup() {

// Initialize serial communication

Serial.begin(115200);

pmSerial.begin(9600);  // PMS5003 uses 9600 baud rate

Serial.println("\n\n====================================");

Serial.println("Air Quality Monitoring System");

Serial.println("====================================");

// Initialize sensor pins

pinMode(MQ5_PIN, INPUT);

pinMode(MQ135_PIN, INPUT);

// Initialize PMS sensor

pms.passiveMode();     // Set PMS5003 to passive mode

delay(1000);           // Give sensor time to initialize

// Connect to WiFi

Serial.print("Connecting to WiFi: ");

Serial.print(ssid);

WiFi.begin(ssid, password);

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

}

Serial.println();

Serial.print("Connected! IP address: ");

Serial.println(WiFi.localIP());

// Connect to Blynk

Serial.println("Connecting to Blynk server...");

Blynk.begin(BLYNK_AUTH_TOKEN, ssid, password);

Serial.println("System initialized and ready!");

Serial.println("====================================");

}

/**

* Main loop function - runs repeatedly

*/

void loop() {

Blynk.run();  // Process Blynk commands

unsigned long currentMillis = millis();

// Read sensors at specific intervals

if (currentMillis - previousMillis >= interval)

{

previousMillis = currentMillis;

// Read all sensors

readMQ5Sensor();

readMQ135Sensor();

readPMSensor();

calculateAirQualityIndex();

sendDataToBlynk();

// Print readings to Serial for debugging

Serial.println("\n--- Sensor Readings ---");

Serial.print("MQ-5 (LPG, Natural Gas): ");

Serial.print(mq5Value);

Serial.println(" ppm");

Serial.print("MQ-135 (Air Quality): ");

Serial.print(mq135Value);

Serial.println(" ppm");

Serial.print("PM2.5: ");

Serial.print(pm25Value);

Serial.println(" μg/m³");

Serial.print("PM10: ");

Serial.print(pm10Value);

Serial.println(" μg/m³");

Serial.print("Air Quality Index: ");

Serial.print(airQualityIndex);

Serial.println(" (0-100)");

Serial.println("----------------------");

}

}

// Read MQ-5 sensor (LPG, Natural Gas)

void readMQ5Sensor() {

int rawValue = analogRead(MQ5_PIN);

// Convert analog reading to parts per million (ppm)

// Note: This is a simple conversion and may need calibration for your specific sensor

mq5Value = map(rawValue, 0, 1023, 0, 1000);

// You can implement a more accurate conversion formula based on your sensor datasheet

// For example: mq5Value = 10 * pow(((1023.0 / rawValue) - 1), -0.631);

}

//Read MQ-135 sensor (Air Quality: CO2, NH3, NOx, etc.)

void readMQ135Sensor() {

int rawValue = analogRead(MQ135_PIN);

// Convert analog reading to parts per million (ppm)

// Note: This is a simple conversion and may need calibration for your specific sensor

mq135Value = map(rawValue, 0, 1023, 0, 1000);

// You can implement a more accurate conversion formula based on your sensor datasheet

// For example: mq135Value = 10 * pow(((1023.0 / rawValue) - 1), -0.4);

}

/**

* Read PMS5003 sensor (PM2.5 and PM10)

*/

void readPMSensor() {

Serial.println("Requesting data from PMS5003...");

// Request reading from PMS5003 sensor

pms.requestRead();

// Wait for sensor to prepare data (minimum 512ms according to datasheet)

unsigned long startTime = millis();

bool dataRead = false;

// Try to read data with timeout

while ((millis() - startTime) < 3000) {

if (pms.readUntil(pmsData)) {

dataRead = true;

break;

}

delay(10);

}

if (dataRead) {

// Successful reading

pm25Value = pmsData.PM_AE_UG_2_5;  // PM2.5 value in μg/m³

pm10Value = pmsData.PM_AE_UG_10_0; // PM10 value in μg/m³

Serial.println("PMS5003 data read successfully");

} else {

Serial.println("Failed to read from PMS5003 sensor");

// Keep previous values if reading fails

// Alternatively, you could set values to -1 to indicate error

}

}

/**

* Calculate overall air quality index

*/

void calculateAirQualityIndex() {

// Calculate individual scores (0-25 scale for each parameter)

// Higher scores mean worse air quality

float mq5Score = constrain(map(mq5Value, 0, MQ5_THRESHOLD * 2, 0, 25), 0, 25);

float mq135Score = constrain(map(mq135Value, 0, MQ135_THRESHOLD * 2, 0, 25), 0, 25);

float pm25Score = constrain(map(pm25Value, 0, PM25_THRESHOLD * 2, 0, 25), 0, 25);

float pm10Score = constrain(map(pm10Value, 0, PM10_THRESHOLD * 2, 0, 25), 0, 25);

// Invert the score so higher numbers mean better air quality (0-100 scale)

airQualityIndex = 100 - (mq5Score + mq135Score + pm25Score + pm10Score);

// Ensure value is in valid range

airQualityIndex = constrain(airQualityIndex, 0, 100);

}

/**

* Send sensor data to Blynk app

*/

void sendDataToBlynk() {

Blynk.virtualWrite(VPIN_MQ5, mq5Value);

Blynk.virtualWrite(VPIN_MQ135, mq135Value);

Blynk.virtualWrite(VPIN_PM25, pm25Value);

Blynk.virtualWrite(VPIN_PM10, pm10Value);

Blynk.virtualWrite(VPIN_AIR_QUALITY, airQualityIndex);

}

/**

* Blynk app notification handler

*/

BLYNK_WRITE(V5) {  // Button in app to enable/disable notifications

int notifyEnable = param.asInt();

if (notifyEnable) {

if (airQualityIndex < 50) {

Blynk.notify("Warning: Poor Air Quality Detected!");

} else {

Blynk.notify("Air Quality Status: Good");

}

}

}

CHAPTER FIVE

# ANDROID STUDIO

# MAIN ACTIVITY JAVA CODE OF AIR QUALITY INDEX METER SYSTEM

package com.huzaifashahbaz.airqualityindexmetersystem

import android.content.Intent;

import android.os.Bundle;

import android.view.View;

import android.widget.Button;

import android.widget.EditText;

import android.widget.ProgressBar;

import android.widget.TextView;

import android.widget.Toast;

import androidx.annotation.NonNull;

import androidx.appcompat.app.AppCompatActivity;

import com.google.android.gms.tasks.OnCompleteListener;

import com.google.android.gms.tasks.Task;

import com.google.firebase.auth.AuthResult;

import com.google.firebase.auth.FirebaseAuth;

import com.google.firebase.auth.FirebaseUser;

public class MainActivity extends AppCompatActivity {

private EditText emailEditText, passwordEditText;

private Button loginButton, registerButton;

private ProgressBar progressBar;

private TextView titleTextView;

private FirebaseAuth mAuth;

@Override

protected void onCreate(Bundle savedInstanceState) {

super.onCreate(savedInstanceState);

setContentView(R.layout.activity_main);

// Initialize Firebase Auth

mAuth = FirebaseAuth.getInstance();

// Initialize UI elements

emailEditText = findViewById(R.id.emailEditText);

passwordEditText = findViewById(R.id.passwordEditText);

loginButton = findViewById(R.id.loginButton);

registerButton = findViewById(R.id.registerButton);

progressBar = findViewById(R.id.progressBar);

titleTextView = findViewById(R.id.titleTextView);

// Set up login button click listener

loginButton.setOnClickListener(new View.OnClickListener()

{

@Override

public void onClick(View v)

{

loginUser();

}

});

// Set up register button click listener

registerButton.setOnClickListener(new View.OnClickListener()

{

@Override

public void onClick(View v)

{

registerUser();

}

});

}

@Override

public void onStart()

{

super.onStart();

// Check if user is signed in and update UI accordingly

FirebaseUser currentUser = mAuth.getCurrentUser();

if(currentUser != null)

{

// User is already signed in, go directly to AirQualityActivity

startAirQualityActivity();

}

}

private void loginUser()

{

String email = emailEditText.getText().toString().trim();

String password = passwordEditText.getText().toString().trim();

// Validate input

if (email.isEmpty())

{

emailEditText.setError("Email is required");

emailEditText.requestFocus();

return;

}

if (password.isEmpty())

{

passwordEditText.setError("Password is required");

passwordEditText.requestFocus();

return;

}

// Show progress bar

progressBar.setVisibility(View.VISIBLE);

// Authenticate user with Firebase

mAuth.signInWithEmailAndPassword(email, password)

.addOnCompleteListener(this, new OnCompleteListener<AuthResult>()

{

@Override

public void onComplete(@NonNull Task<AuthResult> task)

{

progressBar.setVisibility(View.GONE);

if (task.isSuccessful())

{

// Sign in success, update UI with the signed-in user's information

Toast.makeText(MainActivity.this, "Authentication successful.",

Toast.LENGTH_SHORT).show();

startAirQualityActivity();

}

else

{

// If sign in fails, display a message to the user

Toast.makeText(MainActivity.this, "Authentication failed: " + task.getException().getMessage(),

Toast.LENGTH_SHORT).show();

}

}

});

}

private void registerUser()

{

String email = emailEditText.getText().toString().trim();

String password = passwordEditText.getText().toString().trim();

// Validate input

if (email.isEmpty())

{

emailEditText.setError("Email is required");

emailEditText.requestFocus();

return;

}

if (password.isEmpty())

{

passwordEditText.setError("Password is required");

passwordEditText.requestFocus();

return;

}

if (password.length() < 6)

{

passwordEditText.setError("Password should be at least 6 characters long");

passwordEditText.requestFocus();

return;

}

// Show progress bar

progressBar.setVisibility(View.VISIBLE);

// Create user with Firebase

mAuth.createUserWithEmailAndPassword(email, password)

.addOnCompleteListener(this, new OnCompleteListener<AuthResult>()

{

@Override

public void onComplete(@NonNull Task<AuthResult> task)

{

progressBar.setVisibility(View.GONE);

if (task.isSuccessful())

{

// Sign in success, update UI with the signed-in user's information

Toast.makeText(MainActivity.this, "Registration successful.",

Toast.LENGTH_SHORT).show();

startAirQualityActivity();

} else

{

// If sign in fails, display a message to the user

Toast.makeText(MainActivity.this, "Registration failed: " + task.getException().getMessage(),

Toast.LENGTH_SHORT).show();

}

}

});

}

private void startAirQualityActivity()

{

Intent intent = new Intent(MainActivity.this, AirQualityActivity.class);

startActivity(intent);

finish();

}

}

# XML LINAER LAYOUT OF AIR QUALITY INDEX METER SYSTEM

<?xml version="1.0" encoding="utf-8"?>

<LinearLayout xmlns:android="http://schemas.android.com/apk/res/android"

xmlns:app="http://schemas.android.com/apk/res-auto"

xmlns:tools="http://schemas.android.com/tools"

android:layout_width="match_parent"

android:layout_height="match_parent"

android:orientation="vertical"

android:padding="16dp"

android:gravity="center_horizontal"

tools:context=".MainActivity">

<TextView

android:id="@+id/titleTextView"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:text="Air Quality Monitor"

android:textSize="24sp"

android:textStyle="bold"

android:layout_marginTop="32dp"

android:layout_marginBottom="32dp"/>

<EditText

android:id="@+id/emailEditText"

android:layout_width="match_parent"

android:layout_height="wrap_content"

android:hint="Email"

android:inputType="textEmailAddress"

android:layout_marginBottom="16dp"/>

<EditText

android:id="@+id/passwordEditText"

android:layout_width="match_parent"

android:layout_height="wrap_content"

android:hint="Password"

android:inputType="textPassword"

android:layout_marginBottom="24dp"/>

<Button

android:id="@+id/loginButton"

android:layout_width="match_parent"

android:layout_height="wrap_content"

android:text="Login"

android:layout_marginBottom="16dp"/>

<Button

android:id="@+id/registerButton"

android:layout_width="match_parent"

android:layout_height="wrap_content"

android:text="Register"

android:layout_marginBottom="24dp"/>

<ProgressBar

android:id="@+id/progressBar"

android:layout_width="48dp"

android:layout_height="48dp"

android:visibility="gone"/>

</LinearLayout>

# AIR QUALITY ACTIVITY JAVA CODE

package com.huzaifashahbaz.airqualityindexmetersystem

import android.graphics.Color;

import android.os.Bundle;

import android.os.Handler;

import android.os.Looper;

import android.view.Menu;

import android.view.MenuItem;

import android.widget.TextView;

import android.widget.Button;

import android.content.Intent;

import android.view.View;

import android.net.Uri;

import android.widget.Toast;

import androidx.annotation.NonNull;

import androidx.appcompat.app.AppCompatActivity;

import androidx.appcompat.widget.Toolbar;

import com.google.firebase.auth.FirebaseAuth;

import com.google.firebase.database.DataSnapshot;

import com.google.firebase.database.DatabaseError;

import com.google.firebase.database.DatabaseReference;

import com.google.firebase.database.ValueEventListener;

import java.util.Random;

public class AirQualityActivity extends AppCompatActivity

{

private TextView mq5TextView, mq135TextView, pm25TextView, pm10TextView;

private TextView aqiTextView, aqiStatusTextView;

private Button syncButton, grafanaButton, adafruitButton;

private FirebaseManager firebaseManager;

private AdafruitIOManager adafruitIOManager;

private AQICalculator aqiCalculator;

// Simulate sensor updates with a handler

private final Handler handler = new Handler(Looper.getMainLooper());

private Runnable sensorUpdateRunnable;

@Override

protected void onCreate(Bundle savedInstanceState)

{

super.onCreate(savedInstanceState);

setContentView(R.layout.activity_air_quality);

// Set up toolbar

Toolbar toolbar = findViewById(R.id.toolbar);

setSupportActionBar(toolbar);

// Initialize UI elements

mq5TextView = findViewById(R.id.mq5Value);

mq135TextView = findViewById(R.id.mq135Value);

pm25TextView = findViewById(R.id.pm25Value);

pm10TextView = findViewById(R.id.pm10Value);

aqiTextView = findViewById(R.id.aqiValue);

aqiStatusTextView = findViewById(R.id.aqiStatus);

syncButton = findViewById(R.id.syncButton);

grafanaButton = findViewById(R.id.grafanaButton);

adafruitButton = findViewById(R.id.adafruitButton);

// Initialize managers

firebaseManager = new FirebaseManager();

adafruitIOManager = new AdafruitIOManager();

aqiCalculator = new AQICalculator();

// Set up button click listeners

syncButton.setOnClickListener(new View.OnClickListener()

{

@Override

public void onClick(View v)

{

fetchLatestSensorData();

}

});

grafanaButton.setOnClickListener(new View.OnClickListener()

{

@Override

public void onClick(View v)

{

openGrafanaDashboard();

}

});

adafruitButton.setOnClickListener(new View.OnClickListener()

{

@Override

public void onClick(View v)

{

openAdafruitDashboard();

}

});

// Set up initial data fetch

setupRealtimeUpdates();

// Start simulated sensor updates (for demonstration purposes)

startSensorSimulation();

}

@Override

public boolean onCreateOptionsMenu(Menu menu)

{

getMenuInflater().inflate(R.menu.menu_air_quality, menu);

return true;

}

@Override

public boolean onOptionsItemSelected(MenuItem item)

{

int id = item.getItemId();

if (id == R.id.action_logout)

{

logout();

return true;

}

return super.onOptionsItemSelected(item);

}

private void logout()

{

FirebaseAuth.getInstance().signOut();

Intent intent = new Intent(AirQualityActivity.this, MainActivity.class);

startActivity(intent);

finish();

}

private void setupRealtimeUpdates()

{

DatabaseReference sensorsRef = firebaseManager.getSensorDataReference();

sensorsRef.addValueEventListener(new ValueEventListener()

{

@Override

public void onDataChange(@NonNull DataSnapshot dataSnapshot)

{

if (dataSnapshot.exists())

{

SensorDataModel sensorData = dataSnapshot.getValue(SensorDataModel.class);

if (sensorData != null)

{

updateUI(sensorData);

}

}

}

@Override

public void onCancelled(@NonNull DatabaseError databaseError)

{

Toast.makeText(AirQualityActivity.this, "Failed to load sensor data: " + databaseError.getMessage(),

Toast.LENGTH_SHORT).show();

}

});

}

private void fetchLatestSensorData()

{

// For a real app, this would fetch the latest data from the sensors

// Here we simulate reading sensors and updating Firebase

SensorDataModel sensorData = new SensorDataModel();

// Simulate realistic sensor values

sensorData.setMq5(500 + new Random().nextInt(300)); // 500-800 ppm range

sensorData.setMq135(15 + new Random().nextInt(35)); // 15-50 ppm range

sensorData.setPm25(10 + new Random().nextInt(30)); // 10-40 μg/m³ range

sensorData.setPm10(20 + new Random().nextInt(50)); // 20-70 μg/m³ range

// Calculate AQI based on PM2.5 and PM10

int aqi = aqiCalculator.calculateAQI(sensorData.getPm25(), sensorData.getPm10());

sensorData.setAqi(aqi);

// Update Firebase and AdafruitIO

firebaseManager.updateSensorData(sensorData);

adafruitIOManager.publishData(sensorData);

// Update UI

updateUI(sensorData);

Toast.makeText(this, "Sensor data updated", Toast.LENGTH_SHORT).show();

}

private void updateUI(SensorDataModel sensorData)

{

// Update sensor value displays

mq5TextView.setText(String.format("%d ppm", sensorData.getMq5()));

mq135TextView.setText(String.format("%d ppm", sensorData.getMq135()));

pm25TextView.setText(String.format("%.1f μg/m³", sensorData.getPm25()));

pm10TextView.setText(String.format("%.1f μg/m³", sensorData.getPm10()));

// Update AQI display

int aqi = sensorData.getAqi();

aqiTextView.setText(String.valueOf(aqi));

// Set AQI status text and color based on the index

String status;

int color;

if (aqi <= 50)

{

status = "Good";

color = Color.GREEN;

}

else if (aqi <= 100)

{

status = "Moderate";

color = Color.YELLOW;

} else if (aqi <= 150)

{

status = "Unhealthy for Sensitive Groups";

color = Color.rgb(255, 165, 0); // Orange

}

else if (aqi <= 200)

{

status = "Unhealthy";

color = Color.RED;

}

else if (aqi <= 300)

{

status = "Very Unhealthy";

color = Color.rgb(128, 0, 128); // Purple

} else

{

status = "Hazardous";

color = Color.rgb(128, 0, 0); // Maroon

}

aqiStatusTextView.setText(status);

aqiStatusTextView.setTextColor(color);

}

private void openGrafanaDashboard()

{

// In a real app, this would be the URL of your Grafana dashboard

String grafanaUrl = "https://your-grafana-instance.com/dashboard/air-quality";

Intent browserIntent = new Intent(Intent.ACTION_VIEW, Uri.parse(grafanaUrl));

startActivity(browserIntent);

}

private void openAdafruitDashboard() {

// In a real app, this would be the URL of your Adafruit IO dashboard

String adafruitUrl = "https://io.adafruit.com/username/dashboards/air-quality";

Intent browserIntent = new Intent(Intent.ACTION_VIEW, Uri.parse(adafruitUrl));

startActivity(browserIntent);

}

private void startSensorSimulation()

{

sensorUpdateRunnable = new Runnable()

{

@Override

public void run()

{

// Simulate sensor readings every 30 seconds

fetchLatestSensorData();

handler.postDelayed(this, 30000);

}

};

// Start the initial simulation after a short delay

handler.postDelayed(sensorUpdateRunnable, 1000);

}

@Override

protected void onDestroy()

{

super.onDestroy();

// Remove callbacks to prevent memory leaks

handler.removeCallbacks(sensorUpdateRunnable);

}

}

# ACTIVITY AIR QUALITY INDEX METER SYSTEM

<?xml version="1.0" encoding="utf-8"?>

<LinearLayout xmlns:android="http://schemas.android.com/apk/res/android"

xmlns:app="http://schemas.android.com/apk/res-auto"

xmlns:tools="http://schemas.android.com/tools"

android:layout_width="match_parent"

android:layout_height="match_parent"

android:orientation="vertical"

tools:context=".AirQualityActivity">

<androidx.appcompat.widget.Toolbar

android:id="@+id/toolbar"

android:layout_width="match_parent"

android:layout_height="?attr/actionBarSize"

android:background="?attr/colorPrimary"

android:elevation="4dp"

android:theme="@style/ThemeOverlay.AppCompat.Dark.ActionBar"

app:popupTheme="@style/ThemeOverlay.AppCompat.Light" />

<TextView

android:layout_width="match_parent"

android:layout_height="wrap_content"

android:text="Air Quality Monitoring"

android:textSize="22sp"

android:textStyle="bold"

android:gravity="center"

android:padding="16dp" />

<!-- AQI Display Section -->

<LinearLayout

android:layout_width="match_parent"

android:layout_height="wrap_content"

android:orientation="vertical"

android:background="#EFEFEF"

android:padding="16dp"

android:layout_margin="16dp">

<TextView

android:layout_width="match_parent"

android:layout_height="wrap_content"

android:text="Air Quality Index (AQI)"

android:textSize="18sp"

android:textStyle="bold"

android:gravity="center" />

<TextView

android:id="@+id/aqiValue"

android:layout_width="match_parent"

android:layout_height="wrap_content"

android:text="--"

android:textSize="48sp"

android:textStyle="bold"

android:gravity="center"

android:layout_marginTop="8dp" />

<TextView

android:id="@+id/aqiStatus"

android:layout_width="match_parent"

android:layout_height="wrap_content"

android:text="Unknown"

android:textSize="20sp"

android:gravity="center"

android:layout_marginTop="8dp" />

</LinearLayout>

<!-- Sensor Values Section -->

<LinearLayout

android:layout_width="match_parent"

android:layout_height="wrap_content"

android:orientation="vertical"

android:padding="16dp">

<TextView

android:layout_width="match_parent"

android:layout_height="wrap_content"

android:text="Sensor Readings"

android:textSize="18sp"

android:textStyle="bold"

android:layout_marginBottom="16dp" />

<!-- MQ-5 Sensor -->

<LinearLayout

android:layout_width="match_parent"

android:layout_height="wrap_content"

android:orientation="horizontal"

android:layout_marginBottom="8dp">

<TextView

android:layout_width="0dp"

android:layout_height="wrap_content"

android:layout_weight="1"

android:text="MQ-5 (Natural Gas/LPG):"

android:textSize="16sp" />

<TextView

android:id="@+id/mq5Value"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:text="-- ppm"

android:textSize="16sp"

android:textStyle="bold" />

</LinearLayout>

<!-- MQ-135 Sensor -->

<LinearLayout

android:layout_width="match_parent"

android:layout_height="wrap_content"

android:orientation="horizontal"

android:layout_marginBottom="8dp">

<TextView

android:layout_width="0dp"

android:layout_height="wrap_content"

android:layout_weight="1"

android:text="MQ-135 (Air Quality):"

android:textSize="16sp" />

<TextView

android:id="@+id/mq135Value"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:text="-- ppm"

android:textSize="16sp"

android:textStyle="bold" />

</LinearLayout>

<!-- PM2.5 Sensor -->

<LinearLayout

android:layout_width="match_parent"

android:layout_height="wrap_content"

android:orientation="horizontal"

android:layout_marginBottom="8dp">

<TextView

android:layout_width="0dp"

android:layout_height="wrap_content"

android:layout_weight="1"

android:text="PM2.5:"

android:textSize="16sp" />

<TextView

android:id="@+id/pm25Value"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:text="-- μg/m³"

android:textSize="16sp"

android:textStyle="bold" />

</LinearLayout>

<!-- PM10 Sensor -->

<LinearLayout

android:layout_width="match_parent"

android:layout_height="wrap_content"

android:orientation="horizontal"

android:layout_marginBottom="8dp">

<TextView

android:layout_width="0dp"

android:layout_height="wrap_content"

android:layout_weight="1"

android:text="PM10:"

android:textSize="16sp" />

<TextView

android:id="@+id/pm10Value"

android:layout_width="wrap_content"

android:layout_height="wrap_content"

android:text="-- μg/m³"

android:textSize="16sp"

android:textStyle="bold" />

</LinearLayout>

</LinearLayout>

<!-- Action Buttons -->

<LinearLayout

android:layout_width="match_parent"

android:layout_height="wrap_content"

android:orientation="vertical"

android:padding="16dp">

<Button

android:id="@+id/syncButton"

android:layout_width="match_parent"

android:layout_height="wrap_content"

android:text="Sync Sensor Data"

android:layout_marginBottom="8dp" />

<Button

android:id="@+id/grafanaButton"

android:layout_width="match_parent"

android:layout_height="wrap_content"

android:text="Open Grafana Dashboard"

android:layout_marginBottom="8dp" />

<Button

android:id="@+id/adafruitButton"

android:layout_width="match_parent"

android:layout_height="wrap_content"

android:text="Open AdafruitIO Dashboard" />

</LinearLayout>

</LinearLayout>