ASSIGNMENT #3

Wireless and Mobile Networks Versions

Wireless and Mobile Network

Submitted By:

[Huzaifa Shahbaz]

[21-CE-009]

Submitted to:

Maam Sobia Arshad

Dated:

27thJun,2025

Department of Computer Engineering,

HITEC University, Taxila

# ASSIGNMENT#3

Analyze and compare Wireless and Mobile Networks Versions on the basis of data rate, applications(uses), modulation schemes, power consumptions. Present in a tabular form.

# WIRELESS AND MOBILE NETWORKS COMPARSION

# MOBILE NETWORK GENERATION(CELLULAR)

Table 1: TABLE OF MOBILE NETWORK GENERATION (CELLULAR)

| GENERATION | DATA RATE | APPLICATION /USERS | MODULATION SCHEMES | POWER CONSUMPTION | KEY FEATURES |
| --- | --- | --- | --- | --- | --- |
| 1G | 2.4 kbps | Voice calls only, Analog voice transmission | FM (Frequency Modulation), FDMA (Frequency Division Multiple Access) | High(2-3W) | First mobile phone system, No digital capabilities, Poor voice quality |
| 2G (GSM) | 9.6-14.4 Kbps | Digital voice calls, SMS text messaging ,Basic data services | GMSK (Gaussian Minimum Shift Keying), TDMA (Time Division Multiple Access) | Medium (1-2W) | Digital encryption, Better voice quality, International roaming |
| 2.5G (GPRS) | 56-114 Kbps | Email, Web browsing, MMS messaging, Basic mobile internet | GMSK, Packet switching | Medium (1-2W) | Always-on connectivity, Pay-per-data usage, Bridge to 3G |
| 2.75G (EDGE) | 384 Kbps | Improved web browsing, File downloads, Video messaging ,Mobile TV | 8-PSK (8-Phase Shift Keying), Enhanced GPRS | Medium (1-2W) | 3x faster than GPRS, Better spectrum efficiency |
| 3G (UMTS) | 384 Kbps - 2 Mbps | Video calling, Mobile internet, Streaming media, Mobile TV, GPS navigation | QPSK (Quadrature Phase Shift Keying), W-CDMA (Wideband CDMA), CDMA2000 | Medium-Low (0.5-1W) | Always-on broadband, Global roaming, Multimedia services |
| 3.5G (HSPA) | 5.76-42 Mbps | HD video streaming ,Mobile broadband, Online gaming, VoIP services | 16-QAM, 64-QAM, MIMO technology, Enhanced W-CDMA | Medium-Low (0.5-1W) | Faster data speeds, Improved latency, Better user experience |
| 4G (LTE) | 100 Mbps - 1 Gbps | HD/4K video streaming, Cloud services, Mobile gaming, IoT applications, Video conferencing | OFDMA (Orthogonal Frequency Division Multiple Access), SC-FDMA, 64-QAM, 256-QAM, MIMO | Low (0.2-0.5W) | All-IP network, Low latency, High spectral efficiency |
| 4.5G (LTE-A) | 1-3 Gbps | Ultra HD streaming, Advanced IoT, Real-time gaming, Augmented Reality | Carrier Aggregation, Enhanced MIMO, 256-QAM, 1024-QAM | Low (0.2-0.5W) | Multi-carrier support, Coordinated multipoint, Enhanced performance |
| 5G | 1-20 Gbps | Ultra HD/8K streaming, Autonomous vehicles, Industrial IoT, Virtual/Augmented Reality, Smart cities, Remote surgery | Massive MIMO, Beamforming, mmWave, OFDMA, LDPC coding | Very Low (0.1-0.3W) | Ultra-low latency (<1ms), Network slicing, Edge computing |

# WIRELESS LOCAL AREA NETWORK(Wi-Fi)

Table 2: TABLE OF WIRELESS LOCAL AREA NETWORK(Wi-Fi)

| STANDARD | DATA RATE | APPLICATION /USERS | MODULATION SCHEMES | POWER CONSUMPTION | KEY FEATURES |
| --- | --- | --- | --- | --- | --- |
| 802.11 (Legacy) | 2 Mbps | Basic networking, File sharing, Internet access | DSSS (Direct Sequence Spread Spectrum), FHSS (Frequency Hopping Spread Spectrum) | Medium (5-10W) | First wireless LAN standard, 2.4 GHz band |
| 802.11b | 11 Mbps | Internet browsing, Email, File transfers | DSSS, CCK (Complementary Code Keying) | Medium (3-8W) | Widely adopted, Good range, 2.4 GHz band |
| 802.11a | 54 Mbps | Multimedia streaming, VoIP, Video conferencing | OFDM (Orthogonal Frequency Division Multiplexing) | Medium (3-8W) | 5 GHz band, Less congested, Higher data rates |
| 802.11g | 54 Mbps | Streaming media, Online gaming, File sharing | OFDM, Backward compatible with 802.11b | Medium (3-8W) | 2.4 GHz band, Combines best of a/b, Widespread adoption |
| 802.11n (Wi-Fi 4) | 150-600 Mbps | HD video streaming, Multi-device connectivity, Cloud backup | MIMO, OFDM, 64-QAM | Medium-Low (2-6W) | Dual-band (2.4/5 GHz), Multiple antennas, Channel bonding |
| 802.11ac (Wi-Fi 5) | 433 Mbps - 6.77 Gbps | 4K video streaming, VR applications, High-density deployments | MU-MIMO, 256-QAM, Beamforming | Low (1-4W) | 5 GHz only, Wider channels, Better efficiency |
| 802.11ax (Wi-Fi 6) | 600 Mbps - 9.6 Gbps | 8K streaming, IoT devices, Smart homes, Enterprise applications | OFDMA, 1024-QAM, MU-MIMO, BSS Coloring | Very Low (0.5-3W) | Both 2.4/5 GHz, Target Wake Time, Improved efficiency |
| 802.11be (Wi-Fi 7) | 5.8-46 Gbps | Ultra-high bandwidth apps, Real-time applications, Metaverse/AR/VR | 4096-QAM, Multi-Link Operation, Enhanced MU-MIMO | Very Low (0.5-3W) | 6 GHz band support, Ultra-low latency, Maximum throughput |

# BLUETOOTH STANDARDS

Table 3:TABLE OF BLUETOOTH STANDARDS

| Version | Data Rate | Applications /Users | Modulation Schemes | Power Consumption | Key Features |
| --- | --- | --- | --- | --- | --- |
| Bluetooth 1.x | 721 Kbps | Basic file transfer, Audio headsets, Mouse/keyboard | FHSS, GFSK (Gaussian Frequency Shift Keying) | High (100mW) | 10m range, Point-to-point, Basic security |
| Bluetooth 2.x (EDR) | 2.1 Mbps | Enhanced audio, File transfers, Print services | π/4-DQPSK, 8-DPSK, FHSS | Medium (10-100mW) | Enhanced Data Rate, Better security, Improved pairing |
| Bluetooth 3.x (HS) | 24 Mbps | High-speed transfers, Video streaming, Large file sharing | Wi-Fi radio for data, Traditional BT for control | Variable | Wi-Fi integration, High-speed channel, Power optimization |
| Bluetooth 4.x (LE) | 1 Mbps | Fitness trackers, IOT sensors, Wearables, Beacons | GFSK, Low Energy protocols | Very Low (0.01-0.5mW) | Ultra-low power, Coin cell battery life, Mesh networking |
| Bluetooth 5.x | 2 Mbps | Smart home devices, Industrial IOT, Medical devices, Audio streaming | LE Coded PHY, Improved LE, Enhanced GFSK | Ultra-Low (0.01-1mW) | 4x range, 2x speed, 8x broadcast capacity |

# CONCLUSION:

In conclusion, the  power consumption values represent typical transmission power levels. The data rates represent theoretical maximum speeds that can be attained under optimal conditions. The modulation schemes describe the major digital modulation techniques that are employed. Actual performance can be influenced by environmental conditions, device specifications, and congestion in the network. Power consumption is also affected by the type of device, usage pattern, and implementation efficiency.