EC-341 Digital System Design

BS(CE)-2021

LAB REPORT # 3

Submitted By:

| Reg. No. | Name |
| --- | --- |
| 21-CE-009 | Huzaifa Shahbaz |

Department of Computer Engineering

BS Computer Engineering Program

HITEC University Taxila

|  | Presentation (Format(0.5)+ Title (0.5)+ Conclusion(1) +Procedure(2) +Objective(1)) | Calculation/ Coding | Observation/ Program Results | Total |
| --- | --- | --- | --- | --- |
| Total | 5 | 5 | 5 | 15 |
| Obtained |  |  |  |  |

___________________________

Lab Instructor.

Engr. Fasih Ahmad

Experiment #3

Overview of ZYBO

Objective:

1. Overview of ZYBO.

2.Zynq AP SoC Architecture.

3.What is Constraints File?

4. Lab Tasks.

Software Used:

Xilinx Vivado

Procedure:

The ZYBO (ZYnq BOard) is a feature-rich, ready-to-use, entry-level embedded software and digital circuit development platform built around the smallest member of the Xilinx Zynq7000 family, the Z-7010. The Z-7010 is based on the Xilinx All Programmable System-onChip (AP SoC) architecture, which tightly integrates a dual-core ARM Cortex-A9 processor with Xilinx 7-series Field Programmable Gate Array (FPGA) logic. When coupled with the rich set of multimedia and connectivity peripherals available on the ZYBO, the Zynq Z-7010 can host a whole system design. The on-board memories, video and audio I/O, dual-role USB, Ethernet, and SD slot will have your design up-and-ready with no additional hardware needed. Additionally, six Pmod connectors are available to put any design on an easy growth path. The Zynq 7010 AP SoC offers the following features:                                                        1.650Mhz dual-core Cortex-A9 processor

2.DDR3 memory controller with 8 DMA channels

3.High-bandwidth peripheral controllers: 1G Ethernet, USB 2.0, SDIO

4.Low-bandwidth peripheral controller: SPI, UART, CAN, I2C

5.Reprogrammable logic equivalent to Artix-7 FPGA

i.4,400 logic slices, each with four 6-input LUTs and 8 flip-flops                                                    ii.240 KB of fast block RAM

iii.Two clock management tiles, each with a phase-locked loop (PLL) and mixed-mode clock

manager (MMCM)                                                                                                                          iv.80 DSP slices

v.Internal clock speeds exceeding 450MHz

vi.On-chip analog-to-digital converter (XADC)

Tasks

Task 1:

Write a program which toggles LED using a slide switch.

CODE:

Output:

Conclusion:

In this lab we had design the zybo,by using its hardware connect into the Xilinx Vivado and also we check the working Zybo.