EC-341 Digital System Design

BS (CE)-2021

LAB REPORT # 11

Submitted By:

| Reg. No. | Name |
| --- | --- |
| 21-CE-009 | Huzaifa Shahbaz |
| 21-CE-020 | Muhammad Ibthesam |
| 21-CE-021 | Yawar Ali Malik |

Department of Computer Engineering

HITEC University Taxila

|  | Presentation (Format(0.5)+Title (0.5)+ Conclusion(1)+Procedure(2)+Objective(1)) | Calculation/Coding | Observation/ Program Results | Total |
| --- | --- | --- | --- | --- |
| Total | 5 | 5 | 5 | 15 |
| Obtained |  |  |  |  |

___________________________

Lab Instructor.

Fasih Ahmad

Experiment #11

Implementation of Mealy State Machine

Objective:                                                                                                                  1. State Machine as a Controller                                                                                2. Design of Mealy State Machine                                                                    3.Overlapping and Non-Overlapping                                                                            4. Example                                                                                                                       5. Lab Tasks

Equipment/Software Used:                                                                                   Xilinx Vivado

Procedure:                                                                                                                        A state machine is an abstract way of describing the behavior of a controller. The state machine description reduces the description of any controller to just four things: inputs, outputs, states, and state transition rules. The state machine description contains no details about the implementation of the controller (no flowcharts, programs, schematics, mechanical diagrams, etc.). Instead, a state machine only describes the different states that the controller can be in, the outputs in each state, and what input conditions will cause transitions between states. A state machine description can be used to describe and analyze existing systems and to aid in the design of new systems. The simplified description represented by a state machine allows us to verify proper operation and document the logical operation of a controller before proceeding with detailed design (e.g.writing a program or designing hardware).                                               Design of State Machine:                                                                                        There are four steps to designing a state machine:                                                   1.define the outputs – these are the output signals used by the controller                 2.define the inputs – the input signals provided to the controller                                 3.define all of the possible states that the controller will need to be in and what values the outputs will take for each of these states                                                  4.define the exact input conditions that will cause the state machine to switch from one state to another.                                                                                          Inputs and Outputs:                                                                                                       A controller’s inputs are typically signals from sensors such as switches, thermometers, pressure gauges, and so on. The outputs are signals that drive actuators such as motors, valves, displays, etc. We will restrict ourselves to inputs and outputs that are binary values (represented as 0/1, false/true, or low/high). To simplify the state machine description, each input and output is given an abbreviation, usually mnemonic. For example an input that indicates that a tank is full might be given the label full, an output that turns a drill motor on might be given the label drill.

Tasks

Task #1:                                                                                                                           Execute the code, given in example for overlapping/non-overlapping case.                                                                                                     STATE DIAGRAM:

Truth Table:

| Present State | Input | Next State | Output |
| --- | --- | --- | --- |
| S0 | 1 | S1 | 0 |
| S0 | 0 | S0 | 0 |
| S1 | 1 | S1 | 0 |
| S1 | 0 | S2 | 0 |
| S2 | 1 | S3 | 0 |
| S2 | 0 | S0 | 0 |
| S3 | 1 | S1 | 1 |
| S3 | 0 | S2 | 0 |

Verliog:                                                                                                                      module mealy(

input din,

input clk,

input reset,

output reg y

);

reg [1:0] cst, nst;

parameter S0 = 2'b00,

S1 = 2'b01,

S2 = 2'b10,

S3 = 2'b11;

always@(posedge clk)

begin

if (reset)

cst <= S0;

else

cst <= nst;

end

always @(cst or din)

begin

case (cst)

S0: if (din == 1'b1)

begin

nst = S1;

y=1'b0;

end

else

begin

nst = cst;

y=1'b0;

end

S1: if (din == 1'b0)

begin

nst = S2;

y=1'b0;

end

else

begin

y=1'b0;

nst = cst;

end

S2: if (din == 1'b1)

begin

nst = S3;

y=1'b0;

end

else

begin

nst = S0;

y=1'b0;

end

S3: if (din == 1'b0)

begin

nst = S0;

y=1'b1;

end

else

begin

nst = S1;

y=1'b0;

end

default: nst = S0;

endcase

end

endmodule                                                                                                         module testbench();

reg sequence_in;

reg clock;

reg reset;

wire detector_out;

mealy uut (.din(sequence_in),.clk(clock),.reset(reset),.y(detector_out));

initial

begin

clock = 0;

forever #5 clock = ~clock;

end

initial

begin

sequence_in = 0;

reset = 1;

#10;

reset = 0;

#10;

sequence_in = 1;

#10;

sequence_in = 0;

#10;

sequence_in = 1;

#30;

sequence_in = 0;

#10;

sequence_in = 1;

#10;

sequence_in = 0;

#10;

sequence_in = 0;

#10;

sequence_in = 1;

#10;

sequence_in = 1;

#10

sequence_in = 0;

#10;

sequence_in = 1;

#10;

sequence_in = 0;

#10;

sequence_in = 1;

#10

sequence_in = 0;

end

endmodule

Waveform:

Task #2:                                                                                                                     You are required to write code for a basic sequential state machine using mealy based design, which detects 11011 for the following cases:                                         a. Overlapping                                                                                                                b. Non-Overlapping                                                                                                    STATE DIAGRAM:                                                                                      Mealy machine (Non-Overlapping) of 11011                                                                          Truth Table:

STATE DIAGRAM:                                                                                      Mealy machine (Overlapping) of 11011                                                                                                          VERLIOG CODE:                                                                                               module sequence_detector(

input clk,

input reset,

input data,

output reg detected

);

parameter IDLE = 2'b00;

parameter STATE1 = 2'b01;

parameter STATE2 = 2'b10;

parameter STATE3 = 2'b11;

reg [1:0] state, next_state;

always @(posedge clk or posedge reset) begin

if (reset)

begin

state <= IDLE;

end

else

begin

state <= next_state;

end

end

always @(*) begin

case(state)

IDLE: begin

if (data)

begin

next_state = STATE1;

end

else

begin

next_state = IDLE;

end

detected = 0;

end

STATE1: begin

if (data) begin

next_state = STATE2;

end else begin

next_state = IDLE;

end

detected = 0;

end

STATE2: begin

if (data) begin

next_state = STATE3;

end else begin

next_state = IDLE;

end

detected = 0;

end

STATE3: begin

if (data) begin

next_state = STATE2;

detected = 0;

end else begin

next_state = STATE1;

detected = 0;

end

if (state == STATE2 && data) begin

detected = 1;

end

end

endcase

end

endmodule

module sequence_detector_tb();

parameter CLK_PERIOD = 10; // Clock period in ns

reg clk;

reg reset;

reg data;

wire detected;

sequence_detector uut (

.clk(clk),

.reset(reset),

.data(data),

.detected(detected)

);

always #((CLK_PERIOD/2)) clk = ~clk;

initial

begin

clk = 0;

reset = 1;

data = 0;

#20; // Wait for 20 ns

reset = 0; // Deassert reset

// Test case 1: Non-overlapping pattern

#10; data = 1;

#10; data = 1;

#10; data = 0;

#10; data = 1;

#10; data = 1;

#10; data = 0;

#10; data = 0;

#10; data = 1;

#10; data = 1;

#10; data = 1;

#10; data = 0;

#10; data = 1;

#10; data = 0;

#10; data = 1;

#10; data = 1;

#10; data = 1;

#10; data = 0;

#10; data = 1;

#10; data = 1;

#10; data = 1;

#10; data = 0;

#10; data = 1;

#10; data = 1;

#10; data = 0;

#10; data = 1;

#100;

$finish;

end

endmodule

Waveform:

Conclusion:                                                                                                          In conclusion, we used a thorough behavioral testbench to validate both overlapping and non-overlapping scenarios after implementing a Mealy state machine for pattern identification in Verilog.